Table of Contents

1.	Course Outline	4

1.1.	Course topics	4

1.2.	Learning Objectives	4

2.	Introduction to Waves	5

2.1.	What are waves?	5

2.2.	Wave Characteristics	6

2.3.	Waves in nature	7

2.4.	Wave theories	7

2.5.	Interests in Water waves	8

2.6.	Reading	8

3.	Introduction & Mathematics Review	9

3.1.	Introduction to Mathematics and Fluids Review	9

3.2.	Taylor Series	9

3.3.	Useful Derivative and Integration Techniques	9

	Chain Rule:	9

	Integration by Parts:	10

	Even Odd functions	10

	Liebnitz Rule	10

3.4.	Complex Numbers and Relationships to Trigonometric Functions	12

3.5.	Vector Analysis:	13

	Dot Product	13

	Cross Product	14

3.6.	Vector Differential Operators and the Gradient	15

3.6.1	Gradient	15

3.6.2	Divergence	16

3.6.3	Curl	16

3.7	Line Integrals	17

3.8	Fourier Analysis	20

4.	Basic Hydrodynamics	21

4.1.	Def Fluid	21

4.2.	Mass conservation	21

4.3	Types of Fluid motion	21

4.3.1	Linear Strain	22

4.4	Conservation of Mass	24

4.5	Conservation of momentum	29

4.6	Body forces and Buoyancy	31

4.7	Shear Stresses	32

4.8	Equations of Motion	34

4.9	Vorticity	36

4.10	Velocity Potential and Stream Function	38

4.10.1	Velocity Potential	38

4.3.1	Stream Function	41

4.4	Bernoulli Equation	42

5	Small Amplitude Wave Theory	45

5.1	Introduction to Wave Theory	45

5.2	Intro to linear wave Boundary Value Problem (BVP)	48

5.2.1	Kinematic and Dynamic BC’s	50

5.3	Linearization	58

5.4	Solution of the Linearized BVP for small amplitude water waves	60

5.5	Parameters Derivable from dispersion equation	71

5.6	Standing wave	76

5.7	Progressive Wave :	78

5.8	EXAMPLES:	81

Homework #2:	85

6.	EXAM I	86

7.	Engineering Properties Derivable from Linear Theory	87

7.1	Progressive Waves	87

7.1.1	Particle Velocities	89

7.1.2	Vertical Displacement	91

7.2	Standing Waves	93

7.3	Partial Standing Waves	95

7.4	Uniqueness of solutions	97

7.5	Pressure	99

7.5.1	Fluid pressures under a progressive wave	99

7.5.2	Fluid pressures under a standing wave	100

7.6	Energy and Energy Propagation in progressive waves	101

7.6.1	Potential Energy	101

7.6.2	Kinetic Energy	104

7.6.3	Total Average Energy	107

7.7	Energy and Energy flux	107

7.8	Waves entering shallow water	115

7.8.1	Conservation of waves	115

7.8.2	Wave refraction	119

7.9	Conservation of Energy	121

7.9.1	Shoaling	123

7.9.2	Example	124

7.10	Diffraction	126

8	Wave Breaking	127

8.1	Surf Similarity number – Iribarren Number	127

8.2	Depth limited breaking	129

8.3	Steepness limited Breaking	130

9	Summary of Linear (Airy) Wave theory- Properties	131

10	Wave Statistics and Spectra:	132

10.1	Wave-by-wave analysis	133

10.2	Discrete Wave Statistics:	135

10.2.1	EXAMPLE	136

10.3	Probability	137

10.4	Spectral Analysis	142

10.5	Directional Spectra	149

10.6	Spectral moments	149

Wave Energy Spectrum	151

Details on Six Commonly Used Spectrum	153

Neumann Spectrum	153

Pierson – Moskowitz Spectrum	154

JONSWAP Spectrum	154

Bretschneider Spectrum	155

# Course Outline

A course on water wave mechanics at the undergraduate level poses many challenges.  The first of which is keep students excited while introducing very complicated topics and ideas.  At the engineering level, this text seeks to provide the introductory level concepts and details that will enable the reader to gain an understanding of the physics and mechanisms without diving too deep into the waters of higher order theory.  This text will prepare students to enter the workplace with the tools necessary to succeed.  For those who choose to advance their academic degree, this text provides the foundation for water wave mechanics that can be advanced, built upon and even molded as higher order solutions and ideas are learned.

This text is written in both narrative and in bullet / equation form.  Narratives are provided to offer background and insight into the subject matter.  The bullet and equation format help the student to focus on the pertinent information and serve as a typed notes format for the course.

# Course topics

    - Introduction

    - Mathematics review

    - Hydrodynamics

    - BVP Solutions for water waves

    - Engineering Properties Derivable from Linear Theory

    - Wave Statistics

    - Extra Topics

      - Long Waves

      - Wave Forces

      - Wavemaker Theory

      - Nonlinear Properties for Small-amplitude Waves

# Learning Objectives

We will be learning the fundamentals of water wave theory backed up by the fluid principals learned previously and reinforced during the first potion of the course.   It is important to note that there is not exactly a single unique solution to this problem as assumptions must be made to arrive at the analytical solutions.

- You will learn to pay attention to detail

  - READ THE PROBLEMS THOROUGHLY

- You will learn MatLab

- Bring Calculators to class, I expect you to work along at your desk as I am working on the board and will call on the class for solutions

- I expect that you want to be here

- No eating, phones, computers, games, puzzles…period!

- No cheating

# Introduction to Waves

Waves are the driving force behind shoreline evolution.  The energy that waves release as they end their lives on the beaches of the world, drive the evolution of those shorelines.  In shallow and intermediate depth ecosystems, the movement of water due to the presence of the waves provides critical circulation, cycling nutrients and oxygen, providing the mixing needed to reproduce life in the seas.

For the engineer, the design wave condition is one of the major factors for the coastal and ocean environment.  The second major design factor is the water level.  Together the water elevation and the wave climate will drive any engineering project in the coastal zone.  As we will learn, these two design factors are closely coupled.  Relationships will be developed between water depth and wave characteristics such as wave height, wave speed or celerity, and wavelength.

# What are waves?

    - A wave is a disturbance that propagates through a medium:

1) some source of disturbance

2) a medium that can be disturbed

3) some physical mechanism through which particles of the medium can influence each other.

    - Gravity and surface tension act to keep water surface flat

    - Remains flat until acted upon by a force

- Wind, rock impacts, ship moving in water, gravitational pull of moon and sun, low pressure system

    - Wave size depends on magnitude of force acting on the water body

    - Wave length

- Long waves can be as long as the ocean basin (tides)

  - Gravity is the important restoring force

- Short waves can be less than a centimeter (capillary waves )

  - Surface tension the important restoring force

    - What are waves responsible for?

      - drive sediment motion along the coast

      - flushing of lagoons (estuaries)

      - forces on offshore structures (oil Platforms)

      - resistance to ship motion (wake)

      - forces on ships

# Wave Characteristics

    - Height =H(crest to trough), Period=T (time between consecutive crests), Length=L(crest to crest), water depth=h

      - From these we can calculate other parameters

L

The wave diagram, Figure 1, helps to identify the major variables that we will be dealing with.

crest

crest

z=h+ η

η(x,t)

a=H/2

z=0

H

η(x,t)

trough

h

z=-h

Figure 1: Diagram of a wave

h=still water depth

Relative water depth  hL or kh=2πhL

Wave Steepness HL

Relative Wave Height Hh

Ursell Number Ur=Lh2Hh=L2Hh3  (measure of nonlinearity)

- Airy- (linear wave)

- Stokes- (higher order nonlinear)

- Cnoidal hL<18, Ur>20 (Long Wavelength)

- Solitary (Infinite Wavelength) limiting case of cnoidal

# Waves in nature

Rarely look like what we have drawn

      - Vary in height, period and direction

      - Water surface (free surface elevation (eta)) is the superposition of multiple waves

# Wave theories

    - Airy (1845) linear wave theory

    - Stokes (1847) higher order theory

    - Boussinesq (1872) long wave theory

# Interests in Water waves

    - World War II

    - Military

    - Petroleum Industry

    - Coastal Erosion

    - Shipping

    - Ocean Energy

  - Reading:

Varying levels of complexity

      - Butt and Russell,  Surf Science, University of Hawaii Press, 2002

      - CEM (Coastal Engineering Manual) US Army Corps of Engineers Part II.

      - Holthuijsen, Waves in Oceanic and Coastal Waters, Cambridge Press, 2007

      - Ochi, Ocean Waves the Stochastic Approach, Cambridge Press, 1998

Exercise 1: Define in your own words Freak Wave or Rogue Wave.  Find an example of a freak wave or rogue wave, reference the source and summarize your findings.  Include any interesting information on the circumstances including wave height and damage.

# Introduction & Mathematics Review

  - Introduction to Mathematics and Fluids Review

This course is focused on the physical characteristics of water waves; however in order to understand wave mechanics, we must first understand the physical characteristics of fluid flows.  As physics is applied mathematics, we must be proficient in the mathematics if we are to learn the physics of fluids and waves.  This section of the course is aimed at laying out the essential tools needed to be successful.

# Taylor Series

If a continuous function fx of the independent variable x, is known at some location, x=x0, then it can be approximated at another location on the x-axis, x=x0+∆x, by the Taylor series:

fx0+∆x=fx0+∂fx0∂x∆x+∂2fx0∂x2∆x22!+…+∂nfx0∂xn∆xnn!

Where all the derivatives are taken at x=x0, and “n”, is the desired order of accuracy.  For very small values of ∆x, the terms involving (∆x)n, where n>1, are much smaller than the first two terms on the right hand side.  We “linearize” the Taylor series by assuming ∆x≪1, keeping the first two terms on the right hand side and neglecting the higher order terms (these become our error).  The Taylor series is a very useful and handy mathematical tool (for developing relationships between fluid properties at two closely spaced locations).

# Useful Derivative and Integration Techniques

# Chain Rule:

ddxcosu=(-sinu)dudx

# Integration by Parts:

u dv=uv-v du

# Even Odd functions

    - Even Function: f-x=fx

Cosine is an even function

Function is symmetric about an axis

- Odd Function:  f-x=-fx

Sine is an odd function

Function is asymmetric about an axis (opposite responses on either side of an axis)

# Liebnitz Rule

The Liebnitz Rule allows us to differentiate integrals in which the limits of integration are also functions of one of the independent variables.  Thus, for a function fx,t:

ddtA(t)B(t)fx,t dx =A(t)B(t)ddtfx,t dx+B'tfBt,t-A'tfAt,t

Where the primes denote differentiation.

Example: Let  It=-tt2costxdx

Then by the Liebnitz Rule,

ddtIt=-tt2ddtcostxdx+ddtt2cos(t2)t-ddt-tcos(-t)t

ddtIt=--tt2xsintxdx+2tcos(t2)t--1cos(-t)t

cosis an even function:

ddtIt=--tt2xsintxdx+2tcost3+cost2

=--tt2xsintxdx+2tcost3+cost2

Using integration by Parts:

u dv=uv-v du
u=x
du=1 dx
dv=-sintxdx 
v=1tcostx
--tt2xsintxdx=x1tcostx-1tcostx1dx

∴--tt2xsintxdx+2tcost3+cost2=

x1tcos(tx)-1t cos⁡(tx)(1)dx+2tcost3+cost2

=xtcos(tx)-1t2 sin⁡(tx)|-tt2+2tcost3+cost2
=xtcos(tx)-1t2 sint3-1t2sin-t2+2tcost3+cost2
=xtcos(tx)-1t2 sint3+1t2sint2+2tcost3+cost2
=xtcos(tx)-1t2 sint3+1t2sint2+2tcost3+cost2

# Complex Numbers and Relationships to Trigonometric Functions

    - Circular

Euler’s Formula

    - Hyperbolic

- Hyperbolic sine:

sinhx=ex-e-x2

for x≪1,   sinhx=x;for x≫1,   sinhx=ex2

- Hyperbolic cosine:

coshx=ex+e-x2

for x≪1,   coshx=1;for x≫1,   coshx=ex2

- Hyperbolic tangent:

tanhx=sinhxcoshx=ex-e-xex+e-x

for x≪1,   tanhx=x;for x≫1,tanhx=1

# Vector Analysis:

We will also be referencing Vector Algebra throughout the course.  A vector in three dimensions is shown below along with the x, y, and z unit vectors, i, j, and k respectively:

In a coordinate system we can describe a vector in terms of its components.  In the 3-Dimensional Cartesian Coordinate system, the unit vector (a vector of unit length 1) is denoted, i, j, k,or i, j, k in the (x,y,z) directions.  The vector a can be written:

a=axi+ayj+azk

Where ax,ay,az, are projections of the vector a, along the (x,y,z) axes.

The velocity vector u can be written:

u=ui+vj+wk

# Dot Product (orthogonality)

A measure of the projection of one vector onto another, if the vectors are co-directional then the cosine function is equal to one and the dot product is the product of the magnitudes of the vectors.

If the vectors are perpendicular then the dot product will be zero since there is no component of one vector projected onto the other.

The dot product is a scalar product of vector arguments, and is defined as:

a∙b= a bcosθ

Where θ, is the angle between vectors a and b.

If θ=(2n+1)90° (or 2n+1π2 radians) then a∙b=0. The vectors a and b are perpendicular (orthogonal).  (Neither vector projects onto the other)

The following identities for the unit vectors are readily derived:

i∙i=1		j∙j=1		k∙k=1

i∙j=0		j∙k=0		i∙k=0

The dot product is commutative (i∙j=j∙i)

a∙b=a bcosθ=axbx+ayby+azbz

The dot product is used to determine the projection of one vector onto another (or the orientation of one vector with respect to another).

In general, the projection of a onto b=a∙b|b|=a bcosθb∙b

And the angle, θ, between vectors a and b is found as:

θ=arccosa∙b|a||b|

# Cross Product

The magnitude of the cross product is equal to the area of the parallelogram described by the two vectors.  Zero when the vectors are collinear and at a maximum when the vectors are perpendicular.

The cross product is a vector product of vector arguments.  Also called the vector product, the magnitude is defined as:

a×b=a bsinθ

The direction of the resultant is defined according to the “right hand rule” in a direction perpendicular to the plane defined by vectors a and b.

For unit vectors

i×i=j×j=k×k=0

i×j=k	j×k=i	k×i=j

The Cross product is not commutative,

j×i=-k		k×j=-i		i×k=-j

A useful method for determining the cross product is to use a determinant form:

a×b=i      j      k  ax   ay   azbx   by   bz  =aybz-azbyi+azbx-axbzj+axby-aybxk

If the cross product is zero and the vectors a and b are nonzero then the two vectors must be parallel or collinear.

Note that both the dot product and the cross product operate on vectors with arbitrary orientation, and are referenced to our Cartesian coordinate system.

# Vector Differential Operators and the Gradient

In this section we introduce the vector differential operations, these allow derivatives of scalars and vectors to be taken in three dimensional space.

∇≡ddxi+ddyj+ddzk

Where ∇ is referred to as the gradient or del operator.

# Gradient

The Gradient or gradient vector operating on a scalar field, S, is written as grad S, or ∇S, and is given by:

∇S=ddxi+ddyj+ddzkSx,y,z=∂S∂xi+∂S∂yj+∂S∂zk

The Gradient of a scalar is a vector, and it indicates the direction of maximum change of the scalar field.

∂S∂xi+∂S∂yj+∂S∂zk=S

If we know the scalar value at one point in the domain, then by using the Taylor series we can estimate the value at some nearby location.

fx+∆x,y+∆y,z+∆z=fx,y,z+∂fx,y,z∂x∆x+∂fx,y,z∂y∆y+∂fx,y,z∂z∆z

We can rewrite the right hand side of the equation as:

=fx,y,z+∂f∂xi+∂f∂yj+∂f∂zk∙(∆xi+∆yj+∆zk)

The second term is the gradient of the function f. and the third term is the differential vector ∆r.

=fx,y,z+∇fx,y,z∙∆rx,y,z

# Divergence

By applying the differential operator to a vector using the dot product we obtain the divergence of that vector, a scalar value:

The divergence of a vector field ux,y,zis given by:

∇∙u=∂∂xi+∂∂yj+∂∂zk∙uxi+uyj+uzk=duxdx+duydy+duzdz
=dudx+dvdy+dwdz

The divergence of a gradient, del squared, ∇2,  is a special operator known as the Laplacian.

∇∙∇S=∂2S∂x2+∂2S∂y2+∂2S∂z2=∇2S

This operator will show up again later when we set up the boundary value problem for water waves.

# Curl

If instead we apply the differential operator to a vector using the cross product, we obtain the curl of the vector.

∇×u=∂∂xi+∂∂yj+∂∂zk×uxi+uyj+uzk=i      j      k ∂∂x   ∂∂y   ∂∂zux   uy   uz  
=∂uz∂y-∂uy∂zi+ ∂ux∂z-∂uz∂xj+∂uy∂x-∂ux∂yk

The curl is a measure of the rotation of in a vector field.

The curl of a vector is a vector.

The Divergence of the curl is zero:

∇∙∇×u=∂∂xi+∂∂yj+∂∂zk∙∂uz∂y-∂uy∂zi+ ∂ux∂z-∂uz∂xj+∂uy∂x-∂ux∂yk=0

# Line Integrals

If we have two points in a plane, say the x-y plane, and a vector exists over this plane, a(x,y).  If we integrate the vector a along a contour line that connects the two points, we would anticipate that the value of that integral will depend on that path the contour line takes from one of the points to the other.

F=P0P1u∙dl

We would like to be able to integrate the vector between the two points and have the integral be independent of the path we take.

It turns out that if the integrand is an exact differential than the solution will only be a function of the end points and not the contour path taken between those end points:

F=P0P1dF

If we now impose that:

u∙dl=dF

(or the velocity component in the along contour direction equals dF)

Then we should have independence of path.

Conceptually, if we only add the component of the velocity that is parallel to the contour then the total sum can only be the integrated velocity from point a to the line perpendicular to the velocity passing through point b.

Line integral:  Looking down (Plan view)

y

u

b

a

x

y

b

cosθ=0

dl

dl

u

u

a

cos θ=1

x

Integrating (sum) u∙dl = |u||dl|cosθ, the projection of u onto dl

If Mean flow is || to contour then, cos θ=1

If Mean flow is perpendicular to contour then, cos θ=0

Only the along contour flow component is counted

Mathematically we express this dot product as:

u∙dl=uxdx+uydy=dF and 
dF=∂F∂xdx+∂F∂ydy=∇F∙dl

By inspection we see that independence of path requires:

ux=∂F∂x and uy=∂F∂y or u=∇F

Therefore, since

∂ux∂y-∂uy∂x=∂2F∂y∂x-∂2F∂x∂y=0
∂2F∂y∂x-∂2F∂x∂y=∂2F∂x∂y-∂2F∂x∂y=0

We see that in order for independence of path the curl of u must be zero.

dl

n

u •dl

u

Ѳ1=0

Ѳ2= 90°

n

No fluid crosses the contour  P0-P1  all flow is along the contour

ѳ

Po             --dl--                          P1

u

u • dl =udlcosθ1 = u

u • ndl=undlcosθ2=0   
since n is⊥to path of integration

z                   in an x-z preferred frame, n=  -dzi + dxkdl

x

# Fourier Analysis

Any piecewise continuous function can be represented over a given time interval as a sum of sines and cosines.  The time t is arbitrary and it is assumed that the function periodic over the time period T.

The Fourier series is given as:

ft=n=0∞ancosnσt+bnsinnσt

=a0+n=1∞ancosnσt+bnsinnσt

a0=12π-ππftdt

an=1π-ππftcos⁡(nt)dt

bn=1π-ππftsin⁡(nt)dt

Or in Complex notation:

ft=n=-∞∞Cneint

Cn=12π-ππfte-intdt

Cn=12(an-ibn)

# Basic Hydrodynamics

# Def Fluid

A fluid is a substance that constantly deforms under shear stresses

# Mass conservation

    - Unit parcel of fluid

      - Derive net rate of mass accumulation inside control volume

      - Derive accumulation of mass over time

      - These must be equal

      - Continuity equation

      - Incompressibility of water

      - Conservation of mass equation for an incompressible fluid

# Types of Fluid motion

y

Translation: movement without strain or rotation (velocity gradients are zero)

u dt

Relative motion depends on velocity

t1=t0+△t

Velocity gradients = 0

v dt

t0

x

∂u∂x=0; ∂u∂y=0       etc

- Linear Strain

If ∇•u= 0 , volume is conserved

Fluid is incompressible

Non-divergent flow

1∂∀∂∂t∂∀=∇⋅u

(volume expansion/contraction)

t1

t0

Y

y

Angular strain

t1

x

t0

- Rotation

t1        to

    - Linear Strain

(Beginnings of Mass Continuity)

Consider a purely x directed flow

∀0= dxdydz	t						t=dt

∂∀

∀0

∀0

dy

d∀

∂∀

dx

u(x0)+∂u(x0)∂x∆x

∂u∂xdxdt

dx

u(x0)

x0

x0+dx

The volume increases as the change in velocity times the time change, dt

Change in Volume, d∀= ∂u∂xdxdtdydz

Look at time rate of change per unit volume:

1∆∀∂∂td∀=1∆∀∂u∂x∆x∆y∆z

=1∆x∆y∆z∂udx∆x∆y∆z=∂u∂xfor x direction

Strain is the increase in the length of a side of the fluid as a fraction of the original length, so the change in the length divides by the original length and we are left with,

∂u∂xdt. Now convert this into a rate, divide by dt.

For the x direction, the change in volume is ∂u∂x.  Volume change is equal to the velocity gradient and the velocity gradient is also equal to the linear strain rate in the x direction.

Similarly for y and z directions

# Conservation of Mass

-we rely on principal that mass is neither created nor destroyed. There is continuity in the amount of mass in the system

-We can track the changes of mass within a particular system by accounting for influx and efflux of mass through the boundary

Assume that we know the fluid properties at point (x0,y0,z0).

Let us examine a reference parcel of fluid about point (x0,y0,z0); a cube oriented along the x, y and z axes with side lengths Δx, Δy, Δz.

z

w

ρVAin=

ρQout

ρQin

v

u

I(xo,yo,zo)

∆z

∆y

y

x0-∆x2             ∆x                xo+∆x2

x

The rate at which fluid mass flows in is equal to the velocity components in the x-directions multiplied by the area of the face of the cube ⟘ to flow, times the density of the fluid (ρ)

Solving for the mass inflow at x= xo- ∆x2:

ρxo-∆x2,yo,zo uxo- ∆x2,yo,zo∆y∆z

[kg/m3]                    Q=discharge [m3/s]       =    [kg/s]

Mass rate

Using Taylor series we can write the mass flux on the face of the cube as a function of the fluid properties at the center of the cube

In: ρxo-∆x2,yo,zo uxo- ∆x2,yo,zo∆y∆z=

ρxo,yo,zouxo,yo,zo- ∂(ρ(xo,yo,zo)uxo,yo,zo∂x ∆x2+∂2ρxo,yo,zouxo,yo,zo∂x2(∆x2)22!+H.O.T. ∆y∆z

Set,

O∆x2=∂2ρu∂x2(∆x2)22!

=ρu- ∂ρu∂x∆x2+O∆x2∆y∆z

Out:  at x= xo + ∆x2 we obtain;

=ρu+∂ρu∂x∆x2+O∆x2∆y∆z

By subtracting the mass flow rate out from mass flow rate in we obtain the net flux in the x direction

-∂ρu∂x∆x∆y∆z+higher order terms

Assuming ∆x,∆y,∆z are of same order ; H.O.T. = O∆x2∆y∆z are O∆x4

Following same procedure for y and z directions we obtain the net flux across all six faces:

Mass flux   - ∂ρu∂x+∂ρv∂y+∂ρw∂z∆x∆y∆z+H.O.T. O∆x4

Now consider accumulation of mass over the small time increment ∆t:

Mass in cube at time to:                                                              Mass at time to+∆t:

ρ(to) ∆x∆y∆z                                                                ρ(to+∆t) ∆x∆y∆z

The net mass accumulation in cube over time ∆t:

ρto+∆t-ρto∆x∆y∆z=

{Expand by Taylor series}

ρt+∂ρ∂t∆t-ρt=[∂ρ∂t∆t+H.O.T.]∆x∆y∆z

The time accumulation (increase/decrease) in mass must equal net mass inflow accumulation over time ∆t

∂ρ∂t∆t+HOT∆x∆y∆z= -∂ρu∂x+∂ρv∂y+∂ρw∂t∆x∆y∆z∆t+H.O.T.

kgm3ssmmm

kgm3ms1m [mmms]

Divide by ∆x∆y∆z∆t,  neglect H.O.T., and allow the time increments and the size of the cube to approach zero, we obtain the exact equation:

∂ρdt+∂ρu∂x+∂ρv∂y+∂ρw∂z=0

∂ρ∂t+∇∙ρu=0

To better understand:

Now expand product terms

∂ρ∂t+ρ∂u∂x+u∂ρ∂x+ρdvdy+v∂ρ∂y+ρ∂w∂z+w∂ρ∂z=0

Group terms with ρ

1ρ∂ρ∂t+u∂ρ∂x+v∂ρ∂y+w∂ρ∂z+∂u∂x+∂v∂y+∂w∂z=0

Note:∂ρ∂t+∂x∂t∂ρ∂x+∂u∂t∂ρ∂y+∂z∂t∂ρ∂z+∂∂x+∂∂y+∂∂z∙u=0

=  1ρDρDt +∇∙u  	total or material derivitive

1ρDρDt+∂u∂x+∂v∂y+∂w∂z=0

1s+ms  1m=[1s]= Rate of Strain

1ρDρDt+∇∙u=0

We have shrunk cube to zero, every particle entering the cube is the cube, and as such our results found using the cube approach apply everywhere, this is the continuum hypothesis.

The first term,  1ρDρ∂t governs the change in density of the fluid.

The second term ∇∙u governs the flow of the fluid into control volume

Continuity says that the total flow into a fixed volume must be matched by a change in density. This can only happen if the fluid is compressible.

The Bulk Modulus of water, E=∂P∂ρρ is large

Ewater=2.07 x 109 N/m2

We assume that water in incompressible therefore ρ is constant and DρDt=0

Leaving us with:

∂u∂x+∂v∂y+∂w∂z=0 in order for mass conservation, flow must be ; ∇∙u=0 non divergent

Divergence is zero, divergenceless

We have not eliminated stratified flow or density variations; however we assume that each fluid particle has a constant density as it moves through the flow.

Density variations are a result of mixing heavier/lighter fluids in the flow.

In this course we do not consider density driven flows.

Example:  Show that water is essentially incompressible.

At what depth is there sufficient pressure to compress  H2O such that the density of the water has changed by 1%?

Given:		  Evwater=2*109Pa,

Δρρ=1%=0.01

Find:   depth  (h)

Sol’n:					Ev= ∂p∂ρρ →Δp=EvΔρρ
Δp=2*109 Pa 0.01
Δp=2*107 Pa
ph-p0=2*107Pa

Using gage pressures,  p0=pATM=0gage ∴p0=0

And knowing that, p=ρg h , the formula for hydrostatic pressure (we will learn this soon!!!)

ph-0=2*107Pa
ρg h=2*107Pa

Taking ρ=1000kgm3;g=10 m/s2

1*104kgm2s2  h=2*107Pa

h=2*103Pa m2s2kg

PA =pressure=force per unit area=Nm2=kg m/s2m2

h=2*103kgm m2s2s2m2kg=2*103 m

# Conservation of momentum

Mass conservation governs mass accumulation and depletion, but offers no information on the flow itself. For this we need conservation of momentum, Newton’s Second Law of Motion:

Total force equals the time rate of change of momentum

F=dρudt=ma

Where the acceleration a, is given as:

a=dudt

Where u is a function of x, y, z, and t.  Also x, y, and z are functions of t as well, so we must use the chain rule to accurately represent a.

a=dudt

=∂u∂t+∂u∂x∂x∂t+∂u∂y∂y∂t+∂u∂z∂z∂t

where ∂x∂t=u, ∂y∂t=v, ∂z∂t=w

=∂u∂t+u∂u∂x+v∂u∂y+w∂u∂z

The acceleration is a combination of the local acceleration and advection.

ax=∂u∂t+u∂u∂x+v∂u∂y+w∂u∂z

ay=∂v∂t+u∂v∂x+v∂v∂y+w∂v∂z

az=∂w∂t+u∂w∂x+v∂w∂y+w∂w∂z

So we say:

a=DuDt

Where:

D( )Dt=∂( )∂t+u∂( )∂x+v∂( )∂y+w∂( )∂z=∂( )∂t+(u∙∇)(  )

Is the Material Derivative

We have developed equation for the RHS of the eq. but need the LHS

F=dρudt=ma

To better understand this we examine the forces on a fluid particle.

# Body forces and Buoyancy

Body Forces: act through the center or mass of fluid particle cube:

Gravity, Electro-Magnetic, radiative heat transfer

Inertial body forces: (Coriolis, centrifugal)

Surface forces: act on the surface of the cube:

Pressure and shear stress

Consider a fluid without motion:  static, no shear stress

ΣF=ma=0 ,Static equilibrium

z

Pz

Pz

Px

Px

y

x

We find that the pressure acting on the cube… P=-ρgz

Hydrostatic pressure varies linearly with depth, z

Exercise 2:

- Using your own diagram of a unit cube submerged in a fluid, Show that P=-ρgz

- Show that the equation for the Buoyant force is; Buoyant force:

FB=ρfluidgV

# Shear Stresses

Let’s examine the fluid cube again, this time with both normal and shear forces.

Convention is that the 1st subscript refers to the axis normal to the plane on which the stress is acting, and the 2nd refers to the direction of the stress.

σzz

z

y

x

τzy

τzx

τxz

τxy

σyy

τxy

τyz

τxz

σxx=-P+τxx

σxx=-p+τxx

τyx

σxx

τyx

σyy

τxz

τxy

τyz

τxz

τzy

τzx

σzz=-p+τzz

Each surface has 2 shear stresses, (τij) and a normal stress (σii)

It can be shown that as the cube is shrunk to toward a point taken to be smaller and smaller, we end up with six stresses to consider.

σxx=-p+τxx

τxy

τxz

τyz

σyy=-p+τyy

σzz=-p+τzz

Where:

p≡-(σxx+σyy+σzz)3

Where:

σxx+σyy+σzz=-p+τxx-p+τyy-p+τzz

p=--3p+τxx+τyy+τzz3

Therefore: (τxx+τyy+τzz)=0, though each term does not need to be zero

Conceptually, the volume of the cube cannot change, so and stretching in one direction must be accompanied by a compression in the other direction

p ≡isotropic       px=py=pz=p

τ≡ non-isotropic

Additionally it can be shown through the conservation of angular momentum that:

τxy=τyx;τxz=τzx;τyz=τzy;

We can now sum Forces ΣF in the x-direction and we have the acceleration a so we can examine momentum in the x-direction.

ΣF=ma
ΣFx=mDuDt

# Equations of Motion

We now are familiar with the forces and can sum forces in the x-direction

ΣFx=mDuDt

We sum the surface forces and body forces using Taylor Series to apply forces at surfaces in terms of the value at the center of the cube     x0+∆x2,x0-∆x2

σxx+∂σxx∂x∆x2∆y∆z-σxx-∂σxx∂x∆x2∆y∆z
+τyx+∂τxy∂y∆y2∆x∆z-τyx-∂τxy∂y∆y2∆x∆z
+τxz+∂τxz∂z∆z2∆x∆y+τxz-∂τxz∂z∆z2∆x∆y
+ρ∆x∆y∆zX=ρ∆x∆y∆zDuDt

X refers to any and all body forces which may be present

Cancel and divide by ∆x∆y∆z to obtain:

ρDuDt=∂σxx∂x+∂τyx∂y+∂τzx∂z+ρX

σxx=-P+τxx

SO, in all three coordinate directions we obtain:

DuDt=-1ρ∂P∂x+1ρ∂τxx∂x+∂τxy∂y+∂τxz∂z+X

DvDt=-1ρ∂P∂x+1ρ∂τyx∂x+∂τyy∂y+∂τyz∂z+Y

DwDt=-1ρ∂P∂z+1ρ∂τzx∂x+∂τzy∂y+∂τzz∂z+Z

Most general form of momentum equation; however, also unusable since we do not know the stresses…

We assume:

1. Assume stresses are zero, we know that only Body force is gravity (g) acting in the –z direction, so Z= -g

We obtain the Euler Equations

DuDt=-1ρ∂P∂x;                      DvDt=-1ρ∂P∂y;               DwDt=-1ρ∂P∂z-g

or DuDt=-1ρ∇P+FB

where      FB=-gk

These equations are okay when we can assume μ=0 or inviscid flow

If not, we must use Navier-Stokes Equation, denoted by assuming a relationship between stresses, velocity gradient and viscosity.

τxx=μ∂u∂x;            τxy=μ∂u∂y;                     τxz=μ∂u∂z

Stress= rate of strain

DuDt=-1ρ∂p∂x+μρ∂2u∂x2+∂2u∂y2+∂2u∂z2

N.-S. equation requires viscous, laminar flow.

If turbulent, must use Reynolds averages N-S eq.

# Vorticity

u+∂u∂z∆z2

-θa

∆z

θb

w+∂w∂x∆x2

w

u

(x0,z0)

y

z

x

∆x

If we know the velocities at point (x0,z0), then at a small distance away, ±∆x2,±∆z2 we find the velocities at the particle edge using the Taylor Series expansion.

Except in this case we are looking at the u velocity change not in the x direction but in the z-direction, and the w velocity change not in the z direction but in the x direction

ux0,z0+∆z2=ux0,z0+∂u(x0,z0)∂z∆z2

wx0+∆x2,z0=wx0,z0+∂w(x0,z0)∂x∆x2

The Angular Velocity θa  is equal to the difference in the u velocity between the edge of the particle and the center divided by that distance between the center and the edge.

or  θa=-u+∂u∂z ∆z2-u∆z2=-∂u∂z

θb=∂w∂x

The average rate of Rotation is the average of θa+θb or

θ=θa+θb2 or 12∂w∂x-∂u∂z

This is 12 times the curl for the j component

In general

∇×u=2θ= ω

ω is the vorticity, defined as twice the angular velocity of a rotating fluid particle.

Explore vorticity:

So we take the curl of the Navier-Stokes equation.

∇ ×  DuDt=-1ρ∇p+μρ∇2u+gk

Substitution ∇x u=ω

DωDt=ω∙∇u+ 1ρ2∇ρ ×∇p+μρ∇2ω

Change in vorticity due to flow (vortex stretching)

Vorticity generation due to baroclinic effects

Vorticity generation due to viscous effects

Exercise 2.5: Graduate only: show the above relationship (do the math)

If  ω=0   then   ∇× U=0, we say the fluid is irrotational

DωDt=(ω∙∇)U  for inviscid fluid with no change in density

An inviscid incompressible fluid is incapable of generating vorticity.

If ω=0 it will remain 0 → irrotational

Particles can follow a circular or elliptical path, that is OK, since the particles themselves do not rotate. (think Ferris wheel)

# Velocity Potential and Stream Function

We have arrived at two fundamental statements for inviscid flows:

- ∇ ∙u=0 (Continuity Eq for incompressible flows)

(non-divergent)

- ∇ ×u=0 (Vorticity = 0)

(Irrotational)

# Velocity Potential

The second statement GUARANTEES that the integral between two points above a plane is independent of path

recall from line integrals:

u∙dl=uxdx+uydy=dF and 
dF=∂F∂xdx+∂F∂ydy=∇F∙dl
u∙dl=∇F∙dl

and this holds if ∇ ×u=0

We can specify the velocity u as the gradient of a scalar, -∇Φ.  This is because the curl of a gradient is identically zero.

∇ ×-∇Φ=0

This scalar function is the VELOCITY POTENTIAL, Ф, and we can conveniently represent the vector u in terms of a scalar function, -∇Ф, (it is easier to work with scalars that with vectors).

Since 2. We know that there exists a scalar function Φ such that

-Φ=-dΦ=u∙dl=udx+vdy+wdz

u=-dΦ = -∇Φ

u=-∂Φ∂x;v=-∂Φ∂y;w=-∂Φ∂z

u∙∂l=u∂x+v∂y+w∂z=-∂Φ

-dΦ=udx+vdy+wdz

=-∂Φ∂xdx+-∂Φ∂ydy+-∂Φ∂zdz

Taking the Divergence of u, we  get:

Recall ∇∙u=0; incompressible continuity

∇∙u=0=∇∙-∇Φ=-∇2Φ=0

This the Laplace Equation, which governs many phenomena (tidal motion, magnetic fields, electrostatics, ground water)

Φ is defined for inviscid, irrotational flows. The flow can be compressible, though we will not address compressible flows.

Φ, a scalar field whose gradient is the velocity. We represent the velocity vector field with a scalar field.

At every location we have a scalar value

| 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 5 | 5 | 5 | 5 | 5 | 2 |
| 2 | 5 | 10 | 10 | 10 | 5 | 2 |
| 2 | 5 | 10 | 20 | 10 | 5 | 2 |
| 2 | 5 | 10 | 10 | 10 | 5 | 2 |
| 2 | 5 | 5 | 5 | 5 | 5 | 2 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 |

Think of a topographic map. You could define topography/ bathymetry by vectors that (in 3 space) specify how steep and in what direction the land/sea floor is or we can give a scalar value a height (reference to some datum).

# Stream Function

A 2D concept can be used for 3D flows that are uniform in 1D

A function perpendicular to the velocity potential

Ψ=P0P1u∙n dl                           dl=|dl|

Ψ is the amount of fluid crossing the contour (perpendicular)  between points P0 and P1

solving for normal vector n and substituting into our equation:

Ψ=P0P1(-udz+wdx)

Again, for independence of path, the integrand must be exact differential

w= ∂Ψ∂x  ;  u=-∂Ψ∂z

Recall; for independence of path we need the curl to be zero;

∂fx∂z-∂fz∂x=0

But in our case  fx= w, and fz= -u

So,

∂w∂z+∂u∂x=0

which we remember from continuity

# Bernoulli Equation

The Laplace Equation, -∇2Ф, and the Euler Equations govern particle velocity (u,v,w) and pressure (p).  Now that we have collapsed the 3 velocity components into a scalar function, Ф, we have only 2 unknowns and 4 equations.

We will show that the Euler Equations can be collapsed into one fundamental equation and 2 redundant equations.

Start with the Euler Equations:

∂u∂t+u∂u∂x+v∂u∂y+w∂u∂z+1ρ∂P∂x=0

∂w∂t+u∂w∂x+v∂w∂y+w∂w∂z+1ρ∂P∂z=-g

Neglect the y terms for simplicity

Substitute u=∇Φ into Euler Equation….

and from irrotationality  ∂w∂x-∂u∂z=0 ;

∂u∂z=∂w∂x

In X:

∂u∂t+u∂u∂x+w∂u∂z+1ρ∂P∂x=0

∂∂tu+u∂∂x+w∂∂zu+1ρ∂P∂x=0

∂∂t-∂Φ∂x+-∂Φ∂x∂∂x+-∂Φ∂z∂∂z-∂Φ∂x+1ρ∂P∂x=0

∂∂t-∂Φ∂x+-∂Φ∂x∂∂x-∂Φ∂x+-∂Φ∂z∂∂z-∂Φ∂x+1ρ∂P∂x=0

∂∂t-∂Φ∂x+-∂Φ∂x∂∂x-∂Φ∂x+-∂Φ∂z∂∂x-∂Φ∂z+1ρ∂P∂x=0

∂∂tu+u∂u∂x+w∂w∂x+1ρ∂P∂x=0

Need chain rule: 
12∂∂xu2=22u∂u∂x and 12∂∂xw2=22w∂w∂x

Or in terms of Velocity potential:

12∂∂x(-∂Φ∂x)2=22∂Φ∂x∂2Φ∂x2

∂∂tu+12∂∂xu2+12∂∂xw2+1ρ∂P∂x=0

∂∂x-∂Φ∂t+ 12∂∂x-∂Φ∂x2+ 12∂∂x-∂Φ∂z2+1ρ∂P∂x=0

∂∂x-∂Φ∂t+12∂∂x-∂Φ∂x2+-∂Φ∂z2+1ρ∂P∂x=0

IN X:

∂∂x-∂Φ∂t+12-∂Φ∂x2+-∂Φ∂z2+Pρ=0

IN Z :

∂∂z-∂Φ∂t+12-∂Φ∂x2+-∂Φ∂z2+Pρ= -g

Now use the inverse chain rule to modify the product

Integrate over x and z

And substitute in u=-∂Φ∂x       and w=-∂Φ∂z

-∂Φ∂t+12u2+w2+Pρ=C'(z,t)
where C'z,t is a function of z and t only (not a function of x)

-∂Φ∂t+12u2+w2+Pρ=-gz+C(x,t)

where Cx,t is a function of x and t only (not a function of z)

C'z,t=-gz+Ct

No x in either of the constants of integration only time and z

-∂Φ∂t+12u2+w2+Pρ+gz=Ct

This is the unsteady form, C(t),  of the Bernoulli Equation.  For steady flow, C(t) = Constant.

Bernoulli Equation is a relationship between the fluid pressure, particle elevation and velocity potential at one location in the flow fluid pressure, particle elevation and velocity potential at another location in the flow.

We have :

Continuity + Euler Equations (u,v,w,p)

And

Laplace Equation and Bernoulli Equation (Ф,p)

Both sets are equally comprehensive description of the flow field for irrotational, inviscid flows.

Homework  #1

Dean&Dalrymple: 2.2, 2.3 plot in Matlab, 2.6, 2.7, 2.8 plot in Matlab, 2.10 plot in Matlab , 3.6

Extra Credit: 2.9

# Small Amplitude Wave Theory

# Introduction to Wave Theory

Waves come in various sizes, shapes and speeds depending on the force generating them.

Wind waves, waves generated by rocks or boats, will all have a different shape during generation.

Once the waves have been generated and leave the region of forcing, the generation zone, they are free to propagate at the speed dictated by their physics and they share some of the same characteristics.  For every wave, whether generated by wind, rock, boat, or other impulse process, the restorative force is gravity.  These are called “Gravity Waves”.

We will talk about two types of waves:  “wind sea” and “swell”

Wind Sea is the term used to describe waves that are still under the influence of the winds.  The size of the wind generated waves (seas) is dependent on the distance over which the wind is blowing or “fetch” and the time or “duration” of the wind.

We say that the seas are “fetch limited”, “duration limited” or “fully developed”

In the region of generation, the wave speed and direction are closely coupled to the wind.

As these waves travel, propagate, out of the fetch /generation region they become swell seas.  The waves disperse both circumferentially and radially.

The radial dispersion is a function of the speed of propagation, as waves move away radially from the zone of generation.  Waves segregate themselves into waves of similar wavelengths and periods.

As the waves leave the generation zone the crest length grows as the circumference of the disturbance increases.

We know that speed is equal to distance traveled divided by the time it takes to travel that distance.

The speed of travel is:  ∆distance∆time=wave lengthwave period

C=L/T

For waves, the longer waves (waves with a larger wavelength L) will travel faster.

Some coasts are more subject to swell than others: West coast USA vs. GOMEX or East Coast (why?)

The reason is the distance from the coast to the generating zone can be larger for the west coast, thereby allowing more dispersion.  For the east coast, the distance between the generating storms and the coast is usually closer and the waves do not have time to sort themselves before reaching land.

In this class we will be focused on the swell waves, or waves that are no longer affected by generating forcing.  We will be concentrating on waves in water depths shallow enough to affect the characteristics.   Measuring the deep water wave characteristics, we will learn how to predict what the wave climate will be at the coast.

Below is a figure identifying the important features of a wave:

L

crest

crest

z=h+ η

η(x,t)

a=H/2

z=0

η(x,t)

H

trough

h

z=-h

L: wavelength = distance between successive crests (or troughs)

H: wave height = distance from crest to trough

A: wave amplitude, ½ wave height

T: wave period = time required for two successive wave crests to pass a stationary point

C: celerity or phase speed = C=L/T

h: water depth

η(x,t): water level elevation

z: vertical reference dimension

# Intro to linear wave Boundary Value Problem (BVP)

Development of Water Wave theory

In Nature, as water waves propagate, they travel through a medium of viscous fluids and over bottoms with variable permeability and roughness. Our simplifications of the nature of fluid flow however tend to be surprisingly successful for the majority of wave conditions.

The reason is due to the fact that the effects of frictional shear stresses at the bottom are limited to a thin layer near the bottom, ‘bottom boundary layer.’

For typical water waves the boundary layer is θ(3mm)

u

S= boundary layer

In fact, energy losses due to friction with the bottom (for most cohesionless sediment) (sand) are very small relative to energy lost due to breaking. On the other hand, bottom friction effects are cumulative, effects over 100 km of continental shelf will affect the near shore wave climate.

For now we make use of our assumptions and results developed previously to develop the theory of small amplitude waves.

In order to pose our Boundary Value problem we need a governing equation and associated boundary conditions describing our domain.

We could use either the Euler equation approach or the approach of the Laplace Equation. We will use the Laplace Equation as it is one equation from which other properties can be described.

Limiting ourselves to (x,z,t) for simplicity (we will later expand to x,y,z,t) we can define our domain as:

Surface boundary conditions

z

Z=η(x,t)

x

Lateral boundary conditions

Governing eq.     ∇2Φ=0

Lateral BC’s

z=-h

x=L

x=0

Bottom boundary conditions

One wavelength x[0 L]

By assuming irrotational motion u=∇∅ and incompressibility ∇∙u=0

Our governing equation is:

∇2∅=0       -h≤z≤η(x,t)

∂2∅∂x2+∂2∅∂y2+∂2∅∂z2=0

for our boundary conditions we need:

- Kinematic BC’s: these relate fluid motion to the motion of the boundary surface

- Dynamic BC’s: these relate the pressure on the ‘free’ surface of the fluid to the deformation of the boundary in response to the pressure

- Lateral BC’s: these define the fluid behavior at the lateral boundaries

# Kinematic and Dynamic BC’s

      - Kinematic BC:  we first define our boundary as a function FB such that

FB(x,z,t)=0  , we are neglecting y for now

(it can be shown that any equation can be rewritten as a sum (or difference) equal to zero)

The KBC is given as…

u∙n=-∂F∂t∇F  on surface F(x,z,t) = 0

∇F=∇F∙∇F=∂F∂x2+∂F∂z2

There are 2 surfaces where the KBC apply. The ‘free surface’ (KFSBC) and the bottom (KBBC or BBC)

At the bottom, since we assume inviscid flow, we have what is termed a ‘free slip’ boundary condition, as opposed to a ‘no slip’ condition for viscous flows.

The Free slip condition states that

-all velocities at the bottom are tangential to the bottom or

-No normal flow occurs at the bottom

So if we find the unit normal vector to boundary

n=∇F∇F ,  then        u∙n=0 at the bottom

FB(x,z,t)=z+h(x)

Since we have defined our domain such that z=-h(x), at the bottom our function, FB , is given by;

x

z

n=∇F|∇F|

h(x)

Applying our KBBC to FB(x,z,t)=z+h(x)

u∙n= u∙∂h∂xi+ 1k∂h∂x2+1=0

u∂h∂x+w=0 ; on z=-hx

w=-u∂h∂x

For a flat bottom ∂h∂x=0 and we obtain w = 0 as expected.

For a sloping bottom, the velocity is allowed to be tangential.

u

wu=-∂h∂x

w

u

Alternatively,

If we choose a point, pr,t, on a surface, FBr,t,  its motion is described by bottom velocity ub .

If we take FB at some later time, t+∆t, p will have moved r+ub∆t

The equation for the surface at the new location then becomes:

FBr+ub∆t, t+∆t=0  and using Taylor series….

0=FBr,t+dFBdt∆t+∅(∆t)2

Expand derivative dFdt using product (chain rule)

= FBr,t+∂FB∂t+uB∙∇FB∆t+…+∅(∆t)2

If we expand the derivatives in brackets,

∂FB∂t+uB∙∇FB=∂FB∂t+u∂FB∂x+v∂FB∂y+w∂FB∂z

Since FBr,t is defined to be zero, and neglecting Higher Order Terms, then we must have

∂FB∂t+uB∙∇FB=0              on z=-h(x)

Goes to zero for our case of solid nonmoving bottom, no change in F with time

And for the case of the bottom boundary:

FB(x,z,t)=z+h(x)

u∂h∂x+w=0 at z=-hx

-∂∅∂x∂h∂x+ -∂∅∂z=0 at z=-hx

And for a flat bottom   ∂h∂x=0

-∂∅∂z=0 or w=0 ;z=-h(x)

At the surface boundary, we have the Kinematic Free surface BC=KFSBC

Recall: u∙n=-∂F∂t∇F  on surface F(x,z,t) = 0

Again we define the surface Fs(x,z,t)=0 for our domain sketch: z=η(x,t) so:

Fs(x,z,t)= z-η(x,t)

Note that on the surface -dFsdt=dηdt

∇Fs=dηdx+1       ;n=∇Fs|∇Fs|=-dηdxi+ 1kdηdx2+1

u∙n= dηdtdηdx2+1

u∙n=-udηdx+wdηdx2+1=dηdtdηdx2+1

∴dηdt+udηdx-w=0

or w=dηdt+udηdx     on z=ηx,t

Again, we can derive this using the Taylor series expansion on Fs for a short time interval ∆t.

∂FB∂t+u∂FB∂x+v∂FB∂y+w∂FB∂z=0
FBx,z,t= z-ηx,t
-∂η∂t-u∂η∂x+w(1)=0

or w=dηdt+udηdx     on z=ηx,t

The expansion shows more clearly that the surface itself Fs does not change    dFdt=0 (if we move with the surface).

On the ‘free surface’ we have a second BC due to the pressure affects, this is the Dynamic free surface BC, or, DFSBC.

- Dynamic BC

The KFSBC is applied on η, which is initially unknown. We must develop another BC to position the surface.

For weaker waves the pressure on the free surface must be equal to atmospheric pressure, which is the same over the surface due to our assumption of small amplitude, small waves not affected by wind or transitioning pressure variations.

We will use Bernoulli eq. to derive DFSBC

-d∅dt+Pnρ+12u2+w2+gz=C(t)

We can simplify by taking Pn as a gage pressure over the surface such that Pn=0.

-d∅dt+12u2+w2+gz=Ct          z=ηx,t

or-d∅dt+12-d∅dx2+-d∅dz2 +gz=C(t)

To this point we have defined over governing eq. and 3 boundary conditions:

-2 kinematic BC’s

u∙n=-dFdt|∇F|

One for the bottom and one for the ‘free’ surface, and one Dynamic BC for the ‘free’ surface…

-Bernoulli’s Eq. -d∅dt+12u2+w2+gη=0           z=η

It remains to establish Lateral BC’s.

- Lateral BC

We must derive the Lateral BC’s from the problem specification.

The BC will depend on if we have a closed box, or flow in one direction or a moving lateral boundary (i.e. wavemaker, or inflow and outflow).

It is possible that a Kinematic BC must be employed if the lateral boundary is a surface where we have an interaction.

At that point we define the Lateral surface S(x,z,t) as a function of z and t and then implement our KBC formula.

u∙n=dSdt|∇S|

z

x

2a

x=ζt

z=ξx

x=L

Assume the equation ζt=asinσt describes the position of the boundary at the piston.

Then, Sx,z,t=asinσt-x=0 , at the Left Boundary and Sx,z,t=x-L=0 , at the Right Boundary.

For our problem as defined in our BVP, we have a wave, periodic in space and time. Therefore, our lateral LB will be,

∅x,t=∅(x+L,z,t)

∅x,t=∅x,z, t+T

Make L: wavelength, and T: wave Period

Similarly the free surface is periodic and we can put Lateral BC’s on the surface position.

ηx,z,t=ηx+L, z, t

ηx,z,t=ηx,z,t+T

Clearly, the assumption of periodicity does not hold in the presence of a wall or structure

- Our complete BVP on (x,z,t):

Gov. Eq.:

-∇2∅=0               -h≤z≤η

Linear  apply superposition

KFSBC:

∂η∂t+-∂∅∂x∂η∂x=-∂∅∂z    ;      z=η      nonlinear

BBC:

-∂∅∂z=0 ; z=-h   linear

DFSBC:

-∂∅∂t+12∂∅∂x2+∂∅∂z2+gη=0     ; z=η (nonlinear)

Lateral BC:

∅(x,z,t)= ∅(x+L,z,t)

η (x,z,t)=η(x+L,z,t)

∅(x,z,t)= ∅(x,z,t+T)

η(x,z,t)=η(x,z,t+T)

Now solve for ∅ & η

For simplicity, to solve analytically we need to linearize our BC,What do we mean by linear and nonlinear?

-an equation containing products of unknowns (ududx) is referenced as a nonlinear equation.

# Linearization

We need a way to reduce the problem such that…

- Regular physical arguments can be made to the  ‘large’ or ‘smallness’ of terms relative to other terms

- Resulting solution returns at least the gross characteristics of waves as observed; simplification cannot render the solution physically meaningless.

If we assume η is some small deviation from z=0 then we can use Taylor series to take BC’s to the z=0 line (∆z=η)

∆z=η  is small

For the DFSBC:

-∂∅∂t+12 u2+w2 +gz z=η=-∂∅∂t+12 u2+w2 +gzz=0+ ∂∂z[(-∂∅∂t+12u2+w2+gz]ηz=0+H.O.T

η is small, the velocities are small.

Products of η&u, η&w as well as  u2 and w2 are smaller.

So keeping terms linear in η or ∅ at z=0

=-∂∅∂t+12 u2+w2 +gzz=0+ -η∂2∅∂z∂t+η2∂u2∂z+∂w∂z2+ηg∂z∂zz=0

-∂∅∂t+gz+ηg=C,    z=0

Datum C≡0 (pressure at surface)

-∂∅∂t+gη=0      ;       z=0

We now have the Linearized DFSBC, LDFSBC

Bring the Boundary from z=η  to z=0

∆z=η  is small

We do the same for the Kinematic BC

∂η∂t+u∂η∂x-wz=η   = ∂η∂t+u∂η∂x-wz=0+ η∂∂z∂η∂t+u∂η∂x-w  z=0+…=0

All small

Applying same assumptions as before we have:

∂η∂t=w         on z=0
     ∂η∂t =-∂∅∂z  ;z=0

Linearized KFSBC, LKFSBC,  Other BC’s are linear and remain unchanged.

Our complete Linearized BVP on (x,z,t):

Gov. Eq.:

∇2∅=0   ;    -h≤z≤η     linear

LKFSBC:

dηdt=-d∅dz ; z=0  linear

BBC:

-d∅dz=0 ; z=-h   linear

LDFSBC:

d∅dt=gη; z=0 (linear)

Lateral BC: linear

∅(x,z,t)= ∅(x+L,z,t)

η (x,z,t)=η(x+L,z,t)

∅(x,z,t)= ∅(x,z,t+T)

η(x,z,t)=η(x,z,t+T)

Now solve for ∅ & η

  - Solution of the Linearized BVP for small amplitude water waves

    - Step 1:

Using the method of separation of variables, we assume that the solution can be written as a product of functions of each of the variables of interest

T is not period.

∅(x,z,t)=X(x)Z(z)T(t)

product solution

    - Step 2:

For T(t), we know that we need a periodic solution, so pick a simple periodic function, try:

T(t)= sin(at)

T is period.

TO test we first use our temporal LBC :

∅t=∅(t+T)

X(x)Z(z)sin(at)= X(x)Z(z)sin[a(t+T)]

Is this true, or better, when is this true?

When : sin(at)=sin(at+aT)

Using trig identity for sin(a+b)

sin(at+aT)=sin(at)cos(aT)+cos(at)sin(aT)

When is sin(at)=sin(at+aT) true?  When aT=2π

or      a=m2πT        … this is angular frequency, σ

σ=2πT

So:  ∅x,z,t= XxZzsinσt;     where σ=2πT

    - Step 3:

Substitute into Laplace Eqn:

∇2∅=∂2∅∂x2+∂2∅∂z2=0

Linear: superposition OK

X’’Zsin(σt)+XZ’’sin(σt)=0

Cancel sin fcn and multiplying by 1XZ

X''X+Z''Z=0

Depends only on x			Depends only on z

X''X=-Z''Z=-λ

Arbitrary separation constant

So,

X’’+ λX=0
Z’’-λZ=0

A set of ODE’s!

In Sturm-Liouville form.

What can λ be?		Find λ for the nontrivial solution

λ=0

λ<0: λ=-k2

λ>0:λ=k2

Try λ =0

X’’=0  X(x)=Ax+B

Z’’=0 Z(z)=Cz+D

Now we have a possible solution to the Laplace Eq.

Apply a spatial periodic BC to obtain constants:

∅x=∅(x+L)

(Ax+B)(Cz+D)sin(σt)=(A(x+L)+B)(Cz+D)sin(σt)

Ax+B=Ax+AL+B

0=AL A=0

If A=0, then

(0(x)+B)(Cz+D)sin⁡(σt)=(0(x+L)+B)(Cz+D)sin⁡(σt)

B=B

0=0  trivial solution!

λ=0, yields a trivial solution, no help in understanding the problem.

Now Try λ<0:λ=-k2

This also leads a trivial solution.

λ<0→λ=-k2      n=some integer

X’’- k2X=0

X(x)=Aekx+Be-kx

Z’’--k2Z=0   therefore   Z’’+k2Z=0

Z(z)=C cos(kz)+D sin(kz)

Then

∅ =XZsin⁡(σt)

Apply Spatial LBC:

∅x=∅(x+L)

(Aekx +Be-kx)(Ccos(kz)+Dsin(kz)) sin⁡(σt)=(Aekx+L+Be-kx+L)(Ccos(kz)+Dsin(kz)) sin⁡(σt)

Aekx+Be-kx=AekxekL+Be-kxe-kL

0=-Aekx+AekxekL-Be-kx+Be-kxe-kL

0=A(ekxekL-ekx)+B(e-kxe-kL-e-kx)

First Try factoring

0=Aekx(ekL-1)+Be-kx(e-kL-1)

0=Aekx+Be-kx (1ekL-1)(ekL-1)

multiply by ekxekL

0=Ae2kxekL+B (1-ekL)(ekL-1)

0=Aek2x+L-B(ekL-1)(ekL-1)

B=Aek2x+L

no good start over from two lines ago

Aekx+Be-kx=AekxekL+Be-kxe-kL

Aekx+B1ekx=AekxekL+B1ekx 1ekL

Multiply through by ekx

Aekxekx+B=AekxekxekL+B1ekL

Multiply through by ekL

AekxekxekL+BekL=AekxekxekLekL+B

AekxekxekL(1-ekL)=B(1-ekL)

B=Aek2x+L

Then, substitute back in for B

Aekx+Aek2x+Le-kx=Aekx+L+Aek2x+Le-kx+L

Aekx+AekxekxekLe-kx= AekxekL+AekxekxekLe-kxe-kL

Cancel some terms,

Aekx+AekxekL= AekxekL+Aekx

Multiply by e-kx

A+AekL=AekL+A

Trivial!

Again a trivial solution, one more eigenvalue to test.

Try   λ>0: λ=k2

X’’+k2X=0  Xx=Acoskx+Bsinkx
Z’’-k2Z=0  Z(z)=Cekz+De-kz

final solution to try, ∅(x)= ∅(x+L)

[Acos(kx)+Bsin(kx)][Cekz+De-kz]sin(σt)=[Acos(k[x+L])+Bsin(k[x+L])[Cekz+De-kz]sin(σt)

Acoskx+Bsinkx=Acos(kx+kL)+Bsin(kx+kL)

Aside:Trig Identity

cos(kx+kL)=cos(kx)cos(kL) – sin(kx)sin(kL)

sin(kx+kL)=sin(kx)cos(kL)+cos(kx)sin(kL)

Acoskx+Bsinkx=Acos(kx)cos(kL) – sin(kx)sin(kL)+Bsin(kx)cos(kL)+cos(kx)sin(kL)

The above is true when

cos(kL)=1  & sin(kL)=0

kL=2nπ

k=2nπL  n=0,1,2…

Focus on 1st harmonic (n=1)

k=2πL      wave number

So the spatially periodic BC’s are satisfied when k=2πL.  Thus the general solution is:

∅=Acoskx+Bsin(kx)Cekz+De-kzsin⁡(σt)

X                                        Z                     T

Since linear in ∅, superposition of solutions is allowed.

∅=Acoskx+Bsin(kx)Cekz+De-kzsinσt=

∅=AcoskxCekz+De-kzsin⁡(σt)+Bsin(kx)Cekz+De-kzsin⁡(σt)=

∅I+∅II

We will focus on ∅I, for now and add back in ∅II later.

- Step 4:

Solve for constants via BC’s

First start with simplest BC, BBC (w=0)

- Apply BBC

w=-∂∅∂z=0    at z=-h

w=-∂∂zAcoskxCekz+De-kzsinσt=0
=-AcoskxkCekz-kDe-kzsinσt =0

Now evaluate at z= -h,

w-h=-AkcoskxCe-kh-Dekhsinσt=0

The above BC must hold for any and all x and t.

We know cos is not zero everywhere and sin is also not zero everywhere, therefore, the only way for this to be true is if:

Ce-kh-Dekh=0
Ce-kh=Dekh

C=De2kh

now substitute into general eq for vel.pot. Φ…

∅=AcoskxDe2khekz+De-kzsinσt

now Force the solution,

∅=AcoskxDe2khekz+ekhekhDe-kzsinσt

Factor out Dekh,

∅=ADekhcoskxekhekz+e-khe-kzsinσt
∅=ADekhcos(kx)ekh+z+e-kh+zsinσt

(**)

recall:         coshkh+z=ek(h+z)+e-k(h+z)2

∅I=2ADekhcoskx[coshkh+z]sinσt
∅II=2BDekhsinkx[coshkh+z]sinσt

constant G               So now we have, general form of vel.pot. in terms of constant G

∅I=GIcoskxcoshkh+zsinσt
∅II=GIIsinkxcoshkh+zsinσt
Type equation here.

- Apply LDFSBC to (**) to obtain G.

At z=0: -∂∅∂t+gη=C(t)

P=0 at surface

Re-arrange to solve for η (free surface)

η=1g∂∅∂t+C(t)g;at z=0

Substitute (**) into BC,

η=1g∂∂t[Gcoskxcoshkh+zsinσt] z=0

η=Gσgcosh(kh)cos(kx)cosσt             at z=0

We need η, by looking at the spatial and temporal BC’s,

η

z=0

H

By inspection we can choose

η=H2cos(kx)cos(σt)

(other choices are available)

Equate  η’s

H2coskxcos(σt)=Gσgcosh(kh)coskxcosσt

∴G=Hg2σcosh(kh)

Standing wave solution!

Substitute G into (**)..

∅=H2gσcosh[kh+z]cosh(kh)coskxsin(σt)                 (***)

Recall the Linearization of the KFSBC: application of this BC will yield a relationship between σ and k

w= ∂η∂t+u∂η∂x   at z=η(x,t)
w-∂η∂t-u∂η∂x=0    (*)

Apply Taylor Series to (*) about z=0,

w-∂η∂t-u∂η∂xz=η    =w-∂η∂t-u∂η∂xz=0   +η∂∂zw-∂η∂t-u∂η∂xz=0

+…=0

Retaining only the linear terms,

w-∂η∂tz=0 =0
or, w=∂η∂t
or-∂∅∂z=∂η∂t

Now, using LKFSBC:

a.k.a. w=∂η∂tz=0       speed of η moving up and down

take our ∅ and η solutions and apply LKFSBC,

-∂∅∂z=∂η∂t, at z=0

-∂∂zHgcoshkh+z 2σcoshkhcoskxsinσt =∂∂tH2coskxcosσt  ; at z=0
-Hgk2σcoshkhcoskxsinσtsinhkh+z= -σH2coskxsinσt    ; at z=0

Cancel and evaluate at z=0;

-gkσcoshkhsinhkh=-σ

Waterdepth (h)

Wavelength (k,L)

Period (σ, T)

Dispersion Equation!

σ= 2π/T

k=2π/L

gksinh(kh)cosh(kh)=σ2

σ2=gktanhkh

# Parameters Derivable from dispersion equation

σ2=gk tanh(kh)

- Knowing σ= 2π/T and k=2π/L

2π T2=g2πLtanh2πhL

L=gT22πtanh2πhL

- Wave speed (celerity)

[σk]

C≡LT=gT22πtanhkh1T

Lo= deep water wave length. Subscript ‘0’ = deepwater

L0=L=gT22π

C≡LT=gT2πtanhkh

C=LoTtanh(kh)

Or

Co=LoT

T does not change

Most useful relationship for costal engineering

Dispersion equation

σ2=gktanhkh

Only one value of k for a given σ and h

for the remainder of the class we will explore the dispersion equation and see what can be gleaned

Let’s look at limits of dispersion

Plot tanh(kh)

For kh≫1  tanhkh=1

For kh≪1 tanhkh=kh

kh≫1  “deep” water

kh≪1   “shallow” water

in deep water, dispersion is:

σ2= gk(1)

2π2T2=g2πL

L0=L=gT22π;

in deepwater the deepwater wavelength

In shallow water, dispersion reduces to:

σ2= gk(kh)

2π2T2=g2π2L2h

L=gh T

LT=gh=CSW

shallow water celerity

Let’s go back to wavelength

We have found that L0=gT22π or 1.56T2m

L=Lotanh(kh) where tanh(kh) gets smaller as kh gets smaller.

As the wave propagates into shallow water, the wavelength gets shorter.

L is at its maximum in deep water

Looking back at C in shallow water, CSW= gh

C=LT=L0Ttanhkh

In deep water C0=gT2π or 1.56T m/s

We can see that the wave will slow down as depth decreases. So the waves slow down and the wavelengths become shorter as depth decreases.

What is ‘deep water’ and ‘shallow water’ when we are talking about waves?

We define deep and shallow basted on the values of kh and the limits of the hyperbolic functions.

Water is ‘deep’ to a wave when either h≫1 or k≫1. Water is ‘shallow’ to a wave when either h≪1 or k≪1.

k=2π/L so k≪1 when L≫1

and k≫1 when L≪1

really long wave: Tsunami, Tide, surge,

always shallow water

really short wave: ripples capillary

always deep water

Formally.

(Remember h is water depth)

Shallow water	kh≤ π10 (0.3141)

2πhL≤π10    or   hL≤120       or    h≤L20

Deep water           kh≥π

2πhL≥π (3.141)

h/L≥1/2    or      h ≥ L/2        h is greater than ½ the wavelength

Exercise: At what depth is a 9 second wave considered in shallow water?

kh≤  π/10≅0.314

2πLh=0.314
h=0.314L2π=0.05 L

Solve for h once we get L

L=gT22πtanh2πhL
L=g922πtanhπ10 or use SW approx. L=g922ππ10
L=126.470.304=38.47m                L=39.73m

~3.3% difference

h=0.05*38.47m=1.92m        or          h=0.05*39.73m=1.98m

or Plug this back into eq and solve for h

L=gT22πtanh2πhL

38.47m=126.47mtanh2πh38.47m

0.304=tanh2πh38.47m

0.314=2πh38.47m

h=1.92m~6ft

# Standing wave

(Draw diagram first)

L

η(L,0)

η(L/2, T/2)

η(0,0)

η(x,0)

η(x,0)

a

H

η(x,T/4)

η(x,T/4)

H

L/2

L/4

L=3π/4

a

η(x,T/2)

η(L/2, 0)

η(x,T/2)

η(L,T/2)

Φ=H2gσcoshk(h+z)coshkhcoskxsinσt

η=H2coskx cosσt

σt=2πTt=2πTT4=π2

What happens in time first?

At time t=0, cos 0=1

At time t= T/4, cos π2=0

σt=2πTt=2πTT2=π

At time t=T/2, cos π =-1

kx=2πLx=2πLL=2π

What about x?

kx=2πLx=2πLL2=π

At x=0, L       cos0=cos 2π=1

At x=L/2       cosπ=-1

kx=2πLx=2πLL4=π2

At x=L/4       cos(π/2)=0

Develop surface displacement for standing wave:

A standing wave is formed when a wave is reflected off of a barrier.

With a standing wave the initial wave form reflects back onto itself off a vertical boundary.

Suppose we have a +x direction wave and a –x direction wave

+x:   ϕI=H2gσcosh(kh+z)cosh(kh)coskxsin(σt)

ηI=H2cos(kx)cos(σt)

-x:   ϕII=H2gσcosh(kh+z)cosh(kh)cos-kxsin(σt)

ηII=H2cos(-kx)cos(σt)

Since ϕ  is linear…superposition is allowed

ϕ=ϕI + ϕII
ϕ=H2gσcoshkh+zcoshkhsinσtcoskx+cos-kx
EVEN FCN    cos(-kx)=cos(kx)  ∴  coskx+cos-kx= 2 cos(kx) 
ϕstand=Hgσcoshkh+zcoshkhsinσtcoskx

ηstand=ηI+ηII
ηstand=H2coskxcosσt+H2cos-kxcosσt
ηstand= H2cos(σt) coskx+cos-kx 
coskx+cos-kx= 2 cos(kx)
ηstand= Hcos(σt) cos(kx)

η=∂Φ∂tz=0

Solution is a standing wave with twice the amplitude!

# Progressive Wave :

Look again at the Standing wave

ϕI=H2gσcoshkh+zcoshkhcoskxsinσt
ηI=H2coskxcosσt
and  σ2=gktanhkh

Now let’s go back and grab the ΦII , sin(kx) solution

This is a second solution to -∇2Φ=0

ϕII=H2gσcoshkh+zcoshkhsinkxcosσt
ηII=-H2sinkxsinσt

Superposition allows us to add or subtract this is from our standing wave solution

ϕ=ϕI-ϕII=H2gσcoshkh+zcoshkhcoskxsinσt-sinkxcosσt
=-H2gσcoshkh+zcoshkhsinkxcosσt-coskxsinσt
=-H2gσcoshk(h+z)cosh(kh)sin(kx-σt)

And for surface elevation,

η=ηI-ηII= H2coskxcosσt+H2sinkxsinσt
=H2coskx-σt
or≡ 
η=1g∂Φ∂t ;at z=0
η=-H2cos(kx-σt)

Let’s explore surface elevation…

L

η(L,0)

η(L/2, T/2)

η(0,0)

η(x,0)

η(x,0)

a

H

η(x,T/4)

η(x,T/4)

H

L/2

L/4

L=3L/4

a

η(x,T/2)

η(L/2, 0)

η(x,T/2)

η(L,T/2)

η=-H2cos(kx-σt)

What happens to surface elevation in time first?

σt=2πTt=2πTT2=π

At time t=0, cos(kx-σt)=cos kx

At time t=T4, coskx-σt=cos (kx-π2)

At time t=T2, coskx-σt=cos⁡(kx- π)

Now let’s see how wave form responds

kx=2πLx=2πLL=2π

in x.  What about x for t=0?

Blue Line

At x=0, L ;cos0=cos 2π=1

kx=2πLx=2πLL2=π

At x=L2;     cosπ=-1

kx=2πLx=2πLL4=π2

At x=L4 ;   cosπ2=0

At x=3L4 ;   cos3π2=0

What about x for t=T4 ;cos(kx-π2)?

Purple Line

At x=0, L ;cos 2π-π2=cos 3π2=0

At x=L2;     cos π-π2=cos π2=0

At x=L4 ;    cos π2-π2=cos0=1

At x=3L4 ;    cos 3π2-π2=cosπ=-1

What about x for t=T2 ;cos(kx-π)?

Red Line

At x=0, L ;cos 2π-π=cosπ=-1

At x=L2;     cos π-π=cos 0=1

At x=L4 ;    cos π2-π=cos-π2=0

At x=3L4 ;    cos 3π2-π=cosπ2=0

The wave form moves horizontally (to the right) in time!!

# EXAMPLES:

- A wave is measured by bottom mounted pressure sensor at a depth of 20 m, and the wavelength is found to be L=60 m

- Is the wave in deep water at h=20m?

At h=20m , L = 60m, L/2= 30 m, so no not in deep water at 20m.

- What is the wavelength at water depth of 8m?

Use dispersion, solve for wave period.

σ2=gktanhkh

2πT2=g2πLtanh2πhL

1T=gL2πtanh2πhL12

T=2πLgtanh2πhL1/2= 6.29s

Get deep water wavelength using period

Lo=gT22π    = 61.85 m

Now use modified dispersion to complete L at 8m.

L8m=Lotanh2πLh=61.85mtanh2πL 8m

Iterate for solution

Starting value for L enter deep water or some reduced value (could try to guess shallow water, Lshallow,LT=ghsw,   L=Tghsw;  ghsw= do not know hsw , so use h=8m  to get C=8.86 ms,  L_guess=6.29*8.86=55.72m 
L8m~48.187m

IS it deep water? Is h/L >  1/2 → 8/48.187=0.166 < 1/2 =0.5 not deep water

Is h/L < 1/20 ? → 0.166 > 0.05 NO (not shallow water either)

kh = 1.04313, tanh(kh) = 0.779  >  0.314 Not shallow water!

- A peak wave period is measured at the end of a pier.

T=12.2 s

A breakwater is planned to be built 200 ft from the pier (this distance does not matter in computation just to say that the site is adjacent to the location of the measurement of the wave period) in 10 m water depth. What is the wavelength expected at the site associated with the peak period?

Use period to obtain Lo

L=Lotanhkh
Lo= gT22π=232.4 m

L10=232.4mtanh2πL10h

=115.4m in 10m water depth

------------------------------------------------------------------------------------------------

- 2011 Tohoku Earthquake; ON March 11, 2011 an earth quake struck off the coast of Japan.  The time of the 9.0 magnitude quake was 05:46 UTC (GMT) or 2:46pm local time or 7:46pm Hawaii Standard Time.

What was the approx. speed of Tsunami was as it traveled away from the source?

Average depth of Pacific Ocean: Depth=15000 ft, or 4570m

Shallow water wave: C=gh=214 m/s  keep in mind that 1 m/s is 2.237 mi/hr

= 478 mi/hr

x≅4000 mi at 478 mi/hr ≅ 8.4 hrs

Let’s check our answer

5:46 GMT quake+8.4 hrs (time to reach Hawaii as calculated above)

At Hawaii, we have 13:37 UTC (GMT) or 3:37 am Pac/Honolulu time

- Semi-diurnal Tide in Atlantic: how fast does the tidal wave propagate?

Atlantic is ~4000 miles from North America to Europe

Semi-diurnal tides peak every 12 hours so 333 mi/hr

Apply what we learned from dispersion eq:

C= gh

If h is 3000m then C= 9.8*3000=171ms=384 mi/hr

## Homework #2:

D&D

3.3

3.4

3.5

3.7

3.9

3.11

3.12

3.13

3.15; in MatLab code up a function for dispersion.

# EXAM I

# Engineering Properties Derivable from Linear Theory

  - Kinematics and Pressure field for progressive waves

  - Kinematics and Pressure field for standing waves

  - Energy and Energy propagation

  - Conservation of waves

  - Refraction, Snells Law, conservation of energy flux

  - Wave Breaking

  - Diffraction

# Progressive Waves

η=H2cos(kx-σt)

ϕ= -H2gσcoshkh+zcoshkh sin(kx-σt)

Properties of Waves	σ2=gktanhkh→ Dispersion Eqn.

u=-∇ϕ

We can rewrite ϕ by forcing the substitution of the dispersion equation:

Let’s look again at Dispersion

σ2=gktanhkh 
σ2=gk sinhkhcoshkh
σ2gkcoshkh=sinhkh
gkσ21coshkh=1sinhkh  
gσkσ1coshkh=1sinhkh
C=σk  ∴gσ1coshkh =σk1sinhkh=C1sinhkh

If we multiply ϕ by σσ=1; and then substitute σ2=gktanhkh

ϕ= -H2σgσ2coshkh+zcoshkhsin⁡(kx-σt)
=-Hgσ2gktanhkhcoshkh+zcoshkh sinkx-σt
=-Hσ2kcoshkh+zsinhkh sinkx-σt
=-H2Ccoshkh+zsinhkhsin(kx-σt)

# Particle Velocities for a Progressive Wave

Horizontal velocity:

u= -∂Φ∂x= +H2gkσcoshkh+zcoshkhcos(kx-σt)

= +H2σcoshk(h+z)sinh(kh)cos(kx-σt)

And acceleration:

∂u∂t= +H2σ2coshk(h+z)sinh(kh)sin(kx-σt)

Vertical velocity:

w=-∂Φ∂z=H2σsinhkh+zsinhkhsin(kx-σt)

And acceleration

∂w∂t=-H2σ2sinhkh+zsinhkhcoskx-σt

At the bottom,  z=-h, k(h+z)=0

cosh(0)=1

sinh(0)=0

w=0 and  u=-H2σ1sinh khcoskx-σt

Both at minimum value. (see figure of cosh / sinh)

w and u increase as z0 from the bottom to the surface

Particle Velocity

(ζ,ξ)

(ζ,ξ) =(zeta ,ksi  )

To find particle position, integrate velocity

ζx,z,t=u(x1+ζ, z1+ξ) dt
ξx,z,t=w(x1+ζ, z1+ξ) dt

Using Taylor Series, this reduces to,

ζ(x,z,t)= u(x1, z1)dt 
ξx,z,t=w(x1, z1)dt

ζ= u(x1, z1)dt=H2gkσcoshkh+z1coshkhcoskx1-σt
=-H2gkσ2coshkh+z1coshkhsin(kx1-σt)
=-H2coskh+z1sinhkhsin(kx1-σt)

# Vertical Displacement

ξ=H2sinhk(h+z1)sinhkhcos(kx1-σt)

A

particle

B

(x1,z1)

ξ

ζ

A=major axis

B=minor axis

ζ=-H2coskh+z1sinhkhsinkx1-σt= -Asinkx1-σt 
ξ=H2sinhk(h+z1)sinhkhcos(kx1-σt)=B cos(kx1-σt)

sin2 +cos2 = 1

ζA2+ξB2=1

The equation of an ellipse

We can learn more by looking at the limits of how these equations behave

Looking at the shallow kh <π10 and deep

-in Deep Water kh>π  ;  hL>12

sinhz→12ez
coshz→12ez

A=H2coskh+z1sinhkh=H2ekhekz1ekh=H2ekz1
B=H2sinhk(h+z1)sinhkh=H2ekhekz1ekh= H2ekz1=A

A=B

This defines circular displacement in deep water and further more at depths deeper than z≤-L2 visibly (≤4%) no movement

-waves do not ‘feel’ the bottom. Presence at bottom does not affect any wave whose wavelength is less than twice the waver depth.

L/T = gh

-Shallow water

A=H2coskh+z1sinhkh=H21kh=HL4πh=HT4πgh
B=H2sinhk(h+z1)sinhkh=H21+z1h

Only B (the vertical displacement) is dependent on z

At the bottom;

z=-h; B=0 , A=HT4πgh

At the surface:

z=0; B=H2 , A=HT4πgh

# Standing Waves

η=HS2coskxcosσt

Φ= HS2gσcoshkh+zcoshkh coskxsinσt

σ2=gktanhkh→ Dispersion Eqn.

# Particle Velocities under a Standing Wave

u = -∂Φ∂x= HS2gkσcoshkh+zcoshkhsinkxsinσt
w = -∂Φ∂z=- HS2gkσsinhkh+zcoshkhcoskxsinσt

Where u and w, are “in phase” and the maximums are at the nodes (horizontal velocity only) and antinodes (vertical velocity only), and HS=2*HP.

# 7.2.2 Particle Displacement under a standing wave

Particle displacements are linear under a standing wave (back and forth motion)

(Draw diagram first)

L

HP

H

HS

L/2

HP

L/4

3L/4

Particles travel a straight line

ζ=-H2coskh+z1sinhkhsinkx1cos(σt)= -Acosσt 
ξ=H2sinhk(h+z1)sinhkhcoskx1cos(σt)=B cos(σt)

ζA2+ξB2=2cos2σt  ≠1

Since ζ is the x-component of the position and ξ is the z-component, we can write the position vector r as:

r=ζi+ξk

with magnitude of:

r=A2+B2 cosσt

The particles move back and forth along a straight line and the line of motion has the slope:

ζξ=-tanhkh+ztankx

At the bottom, z=-h,  tanhkh+z=0, and the trajectories are horizontal as expected and required.

# Partial Standing Waves

As the name implies, when waves are reflected off irregular objects, as is most often the case, the irregularities prevent pure reflection, and we end up with a partial standing wave.

Reflection- Waves are partially reflected by obstacles, bridge waters, and beaches.

Reflected waves are smaller than the incident wave due to dissipation of wave energy.

ηt=Hi2cos(kx-σt)+Hr2cos(kx+σt+ϵ)

Hi>Hr

Ωi= kx-σt wave propogating to hte right
  Ωr=kx+σt+ ϵ;  wave propogating to the left
 where ϵ is the phase lag

Ti=Tr

Using Trigonometric identities, the total surface displacement is:

ηt=Hi2coskx+Hr2coskx+ϵcosσt+ Hi2sinkx-Hr2sin kx+ϵsinσt

ηt=Ixcosσt+Fxsinσt

To find the total surface displacement of ηx, take derivatives and set to 0.

∂ηt∂t= -Ixσsinσt+Fxσcosσt=0

tan(σt)max= F(x)I(x)

If we look at the extreme values for insight, we note:

ηtx= -Hi22+Hr22+Hi Hr 2cos2x+ϵ

(ηt)max=-Hi2+Hr22
(ηt)max=12Hi+Hr   quasi antinodes

(ηt)min=12Hi-Hr   quasi nodes

Hi=(ηt)max+(ηt)min

Hr= (ηt)max- (ηt)min

Reflection coefficient:

kr=HrHi

# Uniqueness of solutions

The question of uniqueness is not 'simple' at all.

One subscriber to the coastal list notes:

“I am trying to calculate the horizontal particle velocity for standing waves.  I have gone through a few references but got different equations.

1) Basic Coastal Engineering (Sorensen 3rd edition 2010)

u = pi*H/T*cosh(ks)/sinh(kd)*sin(kx)*sin(wt)

2) Mechanics of Wave Forces on Offshore Structures (Sarpkaya/Isaacson 1981)

u = 2*pi*H/T*cosh(ks)/sinh(kd)*sin(kx)*sin(wt)

3) Introduction to Coastal Engineering and Management (Kamphuis 2000)

u = 2*pi*H/T*cosh(ks)/cosh(kd)*sin(kx)*sin(wt)

4) Waves in Oceanic and Coastal Waters (Holthuijsen 2007)

u = -2*pi*H/T*cosh(ks)/sinh(kd)*sin(kx)*cos(wt)

s = d + z

Equation 1 and Equation 2 are different by a factor of 2.

Equation 2 and Equation 3 are different on the denominator -> sinh(kd) instead of cosh(kd) Equation 4 is quite different than others.

I have set up an Excel spreadsheet and confirmed that none of the equations give the same results.

Also, for Equations 1 to 3, the term sin(wt) = 0 when t = 0.  Thus, u = 0 for all x when t = 0.  I am somewhat puzzled by this.  It would be greatly appreciated if someone can point me to any discussion on this.  Thanks.”

And Professor Reeve at Swansea responds

…  is necessary to retrace the derivation of linear wave theory. My copy of Lamb's 'Hydrodynamics' from 1906 has a preface in which Lamb states "I have persisted in the use of the reversed sign of the velocity potential, writing u = -dphi/dx, etc., in place of u = dphi/dx, etc. The physical interpretation of the function is thereby greatly improved, and the far-reaching analogy with the magnetic potential is at the same time made more complete. It should be remembered that the prevalent usage had a purely analytical origin, in connection with the notion of 'perfect differentials.' From a physical point of view it is so much more natural to regard the state of motion of a dynamical system, in any given configuration, as specified by the impulses which would start it, rather than those which would stop it, that the change here advocated would seem to require no further justification.!

" {In the above I have typed phi for the velocity potential}. Whether one agrees with this or not is rather irrelevant. However, the convention is important in determining the form of the final solution.”

And he continues;

“variations in solutions due to differing conventions have a long history. Indeed, the definition of the velocity potential is simply the beginning of a trail of conventions, (which, if justified at all are often done so in a rather offhand manner), that lead to a confusing plethora of possible solutions. {If you are able to find a copy of Lamb's book, Chapter IX covers surface waves.}

Most of the variations can be traced to one or more of:

1) Writing the amplitude of the wave as half the wave height (H/2);

2) Assuming the surface purturbation to be of the form cos(..) rather than sin(..);

3) Assuming the argument of the (co)sinusoidal variation to be wt-kx rather than kx-wt;

4) Taking z = 0 at the seabed rather than at the level of the undisturbed fluid;

5) Noting that L/T = c and using the substitution c = (g/w)tanh(kd).

This already gives 2^5 = 32 variations before including more exotic variants! I would not expect any of them to agree with any of the others for specific values of x, z and t. However, they should all be equivalent in that they should describe the propagation of a linear wave of sinusoidal shape. The waves may be cosines or sines, travelling from right to left or left to right etc etc.

If you do not wish to work through the solution from first principles to obtain a self-consistent set of formulae for velocity potential, surface elevation, horizontal velocity etc. then I suggest you select a set of formulae from one of your sources and at least check that they are self-consistent, perhaps with your Excel spreadsheet.

All your four solutions will have u = 0 for all x for certain specific values of t, (for 1-3 this is when t = 0+n2pi, and for 4 it is when t = pi/2 + n2pi, where n is a positive integer). This is correct as in a standing wave there will be instants when the water surface is momentarily in its undisturbed position, although it will be in motion. [A standing wave is simply a fixed cosine waveform in space modulated in a sinusoidal manner with time. Some of the time the modulation is positive, some of the time the modulation is negative, and occasionally it is zero.]

The usual derivation of standing waves uses the superposition of a left-going and a right-going wave. For sake of argument let us say we have two waves of unit amplitude. The superposition of the waves may be written as cos(kx-wt) + cos(kx+wt). Using standard trigonometric relations this can be simplified to 2cos(kx)cos(wt). If on the other hand we have sinusoidal waves then sin(kx-wt) + sin(kx+wt) = 2sin(kx)sin(wt). The solution you quote that does not begin with a 2 looks odd from this perspective - but you will need to check the author's conventions to verify this.

I leave the final word to Horace Lamb who notes, with a certain amount of glee I suspect, that the corresponding solution of the streamfunction is given, (for the conventions he adopts), by

-tanh(ks)tan(kx).phi

“

I include this conversation to illustrate the need to understand and accept the concept of progressive and standing waves and their solutions, and the assumptions and conditions under which those solutions were derived.

# Pressure

The pressure under a wave is the sum of two components, the hydrostatic component and dynamic component.

# Fluid pressures under a progressive wave

In order to invoke pressure we need to use the Bernoulli equation which relates pressures in our flow, the Bernoulli equation can be written in a variety of forms depending on the units of each of the terms that make up the equations (head, m; pressure, Nm2, velocity squared).

-d∅dt+Pρ+12u2+w2+gz=0

From the linear Bernoulli Equation:

P=-ρgz+ρ∂Φ∂t

(Nonlinear: P=-ρgz+ρ2u2+w2+ρ∂Φ∂t)

The first term of the linear equation is the hydrostatic pressure due to the still fluid. The second term is the dynamic pressure due to the motion of the wave.

- Substituting in the expression for Φ:

P=-ρgz+ρgH2coshkh+zcoshkhcoskx-σt

Often the factor coshkh+zcoshkh is referred to as the Pressure response factor Kp(z). Thus:

Kpz=coshkh+zcoshkh

P(z)= ρgηKpz-z or Pz=-ρg(z-ηKpz)

The dynamic contribution of the wave increases the pressure under a crest and reduces it under a trough. However, the dynamic contribution is reduced at the bottom by:

ηKp-h=1coshkh η

The attenuation of the dynamic pressure is a limiting factor in using bottom mounted pressure gages for wave measurement. For any given depth h, wave number k increases with increasing σ. Thus cosh(kh) increases and Kp decreases. Pressure gages usually have a high frequency limit above which waves go undetected.

k=2πL gets big as L gets small

σ=2πT gets big as T gets small

and as σ ↑;k↑or as T↓ ;L↓

as σ ↑;kh↑relative depth gets deeper

Short period waves have small Kp at the bottom; researcher may not be able to pick up the wave using pressure gauges.  For Long period waves, Kp→1

    - Fluid pressures under a standing wave

P=-ρgz+ρ∂Φs∂t

We then substitute in the velocity potential for a standing wave and solve to get;

Pz=-ρg(z-ηsKpz)

Where Kpz=coshkh+zcoshkh , is the same as for progressive wave.

At nodes P=-ρgz

At antinodes the force on the wall is found by integrating the P(z) over depth:

F=-hηPzdz

# Energy and Energy Propagation in progressive waves

Energy is important as it allows us to determine how waves change, determine the power for energy extraction and wave generation.

We remember from physics that there are 2 kinds of energy:

- Potential Energy: 	displacement of free surface against gravity

PE= mgz

- Kinetic Energy: 	water particles in fluid that are moving

KE=12 mv2

Potential Energy

Flat surface

:

Displaced surface

PE minimum

z

∂z

∂x

The differential mass associated with the column of fluid is:

dm=ρ(h+η)dx

And the potential energy associated with that differential column is:

dPE= dmgz; where z= h+η2
dPE= ρg(h+η)22dx

Now if we integrate this energy over one wavelength L, for a wave with height H, and then average the energy over that same wavelength, L, we obtain the wave averaged potential energy.

‘wave averaging’ : 1Lxx+Ldx

Instead of averaging over one wavelength, we could have chosen to average over one wave period, this is termed, phase averaging:

‘phase averaging’ : 1Ttt+Tdt

wave averaged PE=

PE= 1Lxx+Lρg h+η22dx
=ρgLxx+L  h22+hη+η22dx
=ρgLh22L+hxx+Lηdx+xx+L12η2dx

(PLBC tells us that ηx=η(x+L))

η=H2cos(kx-σt)

So xx+Lηdx=0

η2=H24cos2(kx-σt)

Let α= kx-σt

=H24121+cos2α

=ρgh22+ρgLxx+L12H28+12cos2α dx

Again the integral over a complete wavelength (or period), xx+Lcos2α dx=0 ; since cos is periodic.

PE=ρgh22+ρgH216

PE waves

PE due to depth w/o waves

PEwaves =ρgH216

Dependent only on wave height

# Kinetic Energy

We can obtain the equation for the Kinetic energy in much the same fashion.

KE=12 mv2

The differential Kinetic Energy is then:

dKE=12dm(u2+w2)

d(KE)= dm(u2+w2)2=ρ dx dz (u2+w2)2= ρ (u2+w2)2dx dz

Substituting in for u & w

u= H2σcoshk(h+z)sinhkhcos(kx-σt)

w=H2σsinh kh+zsinhkhsinkx-σt

d(KE)=12ρH2σ24(sinhkh)2cosh2kh+zcos2kx-σt+sinh2kh+zsin2(kx-σt)dxdz

Useful Identities:

cosh2x=12cosh2x+12
sinh2x=12cosh2x -12
sin2x=12-12cos2x
cos2x=12+12cos2x

let  q=hh+z, Ω=kx-σt

cosh2qcos2Ω+sinh2qsin2Ω =12cosh2q+12 12+12cos2Ω +12cosh2q -1212-12cos2Ω

=1212cosh2q+1212+1212cos2Ω +1212cosh2qcos2Ω +1212cosh2q-1212+1212cos2Ω -1212cosh2qcos2Ω 
=1212cosh2q+1212cos2Ω +1212cosh2q+1212cos2Ω 
=12cosh2q+12cos2Ω

Substitute back in for q and Ω and into our equation for KE:

=12ρH2σ24(sinhkh)212cosh2kh+z+cos2kx-σtdxdz

We have rid ourselves of the pesky squared terms and now can integrate.

Integrate over depth and average over time or wavelength

KE=116ρH2σ2sinh2kh1Lxx+L-hηcosh2kh+z+cos2(kx-σt)dz dx

let X=116ρH2σ2sinh2kh

Derive KE

σ2= gk tanh(kh)

:

KE=XLxx+L-hηcosh2kh+z+cos2kx-σt dz dx
First integrate over depth

=XLxx+L  -hη≈0cosh2kh+zdz+-hη≈0cos2kx-σt dz dx 
=XLxx+L12ksinh2kh+z |0-h    +cos2kx-σtz|0-h dx
=XLxx+L12k (sinh2kh+0 -sinh2k(h-h))+cos2kx-σt0-cos2kx-σt-hdx 
=XLxx+L12k sinh2kh dx +xx+Lhcos2kx-σt dx

Second integrate over wavelength

=XL12ksinh2kh(x)|x+Lx+hxx+L cos2kx-σt dx
=XL12ksinh2khx+L-x+ 0
=XL12k(sinh2kh) L
=X sinh2kh2k
=116ρH2σ2sinh2kh sinh2kh2k
=ρH216 gktanhkh 2ksinh2khsinh2kh
=ρgH21612sinhkhcoshkhsinh2kh sin2kh
=ρgH216 12sinh2khcoshkhsinhkh
=ρgH216

KE=ρgH216

Alternate derivation:KEunit volume=12pu2+w2             Q=hh+z, q=kh
=12 ρH2σ24sinh2q12cosh2Q+cos2kx-σt

Integrate over depth, then average over time for average KE per unit surface area

KE=116ρH2σ2sinh2q  1T0T-hη(cosh2Q+cos2(kx-σt))dzdt

Constant in time

=ρH2σ216sinh2q1T0T12ksinh2qdT

Use σ2= gk tanh(kh)

=ρgH216sinh2q2sinhqcoshq

KEunit volume=ρgH216

# Total Average Energy

Energy in wave is equally divided between energy contained in difference in water elevation and still water levelenergy due to water movement

We find that PE=KE for our system, therefore the total average energy per unit surface are of a wave is given by

E=PE+KE=ρgH216+ρgH216=18 ρgH2

for all linear waves.

# Energy and Energy flux

E=18ρgH2

kgm3ms2m2≡Nm≡Jm2

For all linear waves.

And the total energy per unit crest width is

EL=18 ρgH2L

kgm3ms2m2m≡N≡Jm

Example: for a wave L=50m, H=1m, calculate the Energy:

E=1263.0 N/m≡Jm2

Or

EL=63,152 N=63kN=6.3 ton force

Relate this to a value that we can understand:

| NAME | DEFINITION | UNIT |
| --- | --- | --- |
| Force | push or pull = mass  acceleration | N=kgms2 |
|  |  | 10kN=1 ton |
| Energy | Capacity to do work = force distance | J=N*m |
| Avg. Energy Per unit wave area |  | J/m2=N/m |
| Avg. Energy Per unit wave crest |  | J/m=N |
| Power | energy  time | J/s=N*m/s |
| Wave energy flux Or Power / unit crest length | Average energy speed of propagation per crest length | Js/m=N/s |

Energy flux: rate of work done due to wave’s rate of transport of wave energy. The rate at which the dynamic pressure is doing work on neighboring column of water

dz

PDu

dI=-hηPDu dz

Integrate PDu over the column of water, with PDu is the dynamic pressure acting in the direction of wave propagation.

The average energy flux (pressure times area times velocity) is:

I=1Ttt+T-hηPDudzdt

‘Phase averaged’ average over one period

I=1Ttt+T-hη [ ρgηcoshkh+zcoshkh ] gσHk2coshkh+zcoshkhcoskx-σtdzdt
Use dispersion eq. to rewrite u

=1Ttt+T-hη  ρgηcoshkh+zcoshkhH2σcoshkh+zsinhkhcoskx-σtdzdt
use definition of  η to rewrite u

=1Ttt+T-hη  ρgηcoshkh+zcoshkhση coshkh+zsinhkhdzdt
=1Ttt+T-hη  ρgη2σ cosh2k(h+z)coshkhsinhkh dzdt
Recall that coshkhsinhkh=12 sinh2kh

=1Ttt+T-hη2ρgη2σsinh2kh  cosh2k(h+z) dzdt
Recall that cosh2k(h+z)=12(1+cosh2kh+z)

=1Ttt+T2ρgη2σsinh2kh -hη12(cosh2kh+z+1) dzdt
for small η, η≈0

=1Ttt+T2ρgη2σsinh2kh 1212ksinh2kh+z+z|0-hdt

where     12[ 12ksinh2kh+z+z]|0-h

Factor out 12k
=1212ksinh2kh+z+2kz|0-h
 =14k [sinh2k h+0+2k0-sinh2kh+-h+2k-h
=14k sinh2kh+0-0+2kh
=14k sinh2kh+2kh

=1Ttt+T2ρgη2σsinh2kh 14k[sinh2kh+2kh] dt

η2is the only variable that is a function of time.
=1T2ρgσsinh2kh 14k[sinh2kh+2kh]tt+TH22 cos2kx-σt dt
Recall trig identity cos2kx-σt=12 (1+cos2kx-σt)

=1T 2ρgσ4k sinh2kh+2khsinh2kh H24 tt+T12 (1+cos2kx-σt) dt

=1T ρg2 C 1+ 2khsinh2khH2412T 
=ρgH2812 C 1+2khsinh2kh
=EC121+2khsinh2kh
Define:

n=121+2khsinh2kh

=ECn
I=ECn

Cn is the speed at which energy is transported in a wave.

Let’s have a look at n:

n=121+2khsinh2kh

For big kh             sinh2kh12 e2kh

2kh12e2kh→0, and n→12  deepwater

For small kh        sinh2kh 2kh

2kh2kh=1, and n=1  shallow water

Energy moves slower than individual waves.

Let’s examine wave travel a bit more and see if we can shed more light on the speed of energy travel Cn

Take 2 waves travelling in the same direction with slightly different wave #’s, k, and frequencies σ.

k1=k-∆k2       σ1=σ-∆σ2

k2=k+∆k2       σ2=σ+∆σ2

By superposition we obtain

η=η1+η2=H12cos(k1x-σ1t)+H22cos(k2x-σ2t)

If H1=H2=H and Recall trig identity for cos(k1x-σ1t)+cos(k2x-σ2t)

=H2 2cos12(k1x-σ1t)+(k2x-σ2t)cos12(k1x-σ1t)-(k2x-σ2t)

k1+k2=2k  ;  k1-k2 =-∆k ;  -σ1+ -σ2 = -2σ ;   -σ1+σ2 = ∆σ

=H2 2cos12(2kx-2σt)cos12(-∆kx+∆σt)

Reduce and factor out ∆k

η=H22cos(kx-σt)cos∆k2-x+∆σ∆kt

η=Hcos(kx-σt)cos∆k2x-∆σ∆kt

Our new wave form, Hcos(kx-σt) consists of waves moving with velocity C=σ/k , these are contained (modulated) in an envelope moving with speed ∆σ/∆k.

z=0

The envelope is our wave group & it moves with celerity of

Cg≡ group velocity= ∆σ/∆k

If we take derivatives of dispersion equation with respect to k

σ2=gktanhkh
ddkσ2=ddkgktanhkh

ddkσ2=ddkgktanhkh+gkddktanhkh
2σdσdk=gtanhkh+gkhsech2kh
dσdk=g2σ tanhkh+khsech2kh
Substitute dispersion equation into denominator

dσdk=gσ2gk tanhkh+khsech2khtanhkh

dσdk=12C1+khsech2khtanhkh

khsech2khtanhkh=khcoshkhsinhkh cosh2kh

dσdk=12C 1+khsinhkh coshkh =C121+2khsinh2kh

sinhkhcoshkh =12sinh2kh

Relates group velocity to wave phase velocity

dσdk=∆σ∆k=nC

∆σ∆k=Cg    ∴ Cg=nC

n=CgC

And energy travels at

F= EnC= ECg      = Group velocity!

As kh changes, wave properties change.

Wave Length     L ↓ decreases

Phase speed      C ↓ decreases

Group velocity  phase velocity Cg =C

Wave height increases shoaling = H ∝ h break

Wave angel decreases to Perpendicular

# Waves entering shallow water

As waves propagate from deep water towards shore, they undergo transformations as the water depth gets shallower.  This transformation process is of particular interest to the coastal engineer.  Using our knowledge of wave transformations enables the engineer to predict the wave conditions at the project site using wave information collected long distances away at some deep water buoy location.

We first convince ourselves that the wave period remains constant as waves propagate into shallow waters.  The conceptual argument rests in the fact that the both the wavelength and wave speed decrease as waves move into shallower waters.  In order for both to decrease, the period must remain constant or change only slightly.  We will develop a more rigorous argument establishing that for steady waves the wave period remains essentially constant.

# Conservation of waves

Let’s look at 2d waves

First, define Ω such that,

η= H2cosΩ

for waves travelling in the x direction,

Ω=(kt-σt) and crests occur when Ω=0, 2π or 2nπ (n is any positive integer)

And η= H2 ≡ max value ≡crest

If we look at waves traveling, the direction of propagation is perpendicular to the wave crest

We call this the wave ray

The unit normal, n, to the direction of travel is equal to the normal vector N divided by the magnitude of the gradient of Ω

N=n∇Ω=k

Define wave number,  k,  as the vector oriented to the direction of wave propagation:

k= ∇Ω=n∇Ω

Where ∇=∇h= ∂∂xi+∂∂yj is the horizontal gradient operator

In 2D, k=kxi+kyj

k=k= kx2+ky2

Where k is the wave number and 
kx= kcosθ               ky=ksinθ

Where θ is the wave angle, angle of incidence, the angle made between the beach normal and the wave direction

ky

θ

kx

k

θ

x

So, we obtain, Ωx,y,t=k∙x-σt

=kxx+kyy-σt

=kcosθ  x+ksinθ y -σt 
=kxcosθ+kysinθ-σt

For the case of waves propagating onshore (shore normal), θ=0 and only the x-term remains

Ωx,y,t=kx-σt

Okay, reduces to 1D equation.

If we plot k as waves propagate

The curved horizontal line everywhere tangent to the wave number, k, is called the wave ‘ray.’

Energy travels along the wave ray

We determine wave frequency, σ from the phase 
Ω=kxcosθ+kysinθ-σt

Taking the time derivative of   Ω we get:

σ=-∂Ω∂t

It follows that

∇σ=∇-∂Ω∂t

∇σ+∂∇Ω∂t=0

recall, k= ∇Ω

then:

∇σ+∂k∂t=0

So a spatial change in frequency must be balanced by the time change in k.

and recall that ∂∇Ω∂t=-∂k∂t=0 ;for steady waves

Since steady waves do not change in time,

∇σ=0

∴ σ does not change in space even as water depth changes

Frequency and therefore period is constant in space for steady waves

-∇σ= ∂k∂t

∂k∂t+∇σ=0      conservation of waves

Just as we know conservation of mass, we have conservation of waves.  Any time change in L must be accompanied by a spatial change in T.

# Wave refraction

To determine how wave θ changes in space let’s look at the curl of k.

k=∇Ω→ ∇×k=∇×∇Ω=0 curl of gradient is always zero

Therefore, Curl of wave number vector field must be zero

∇×∇Ω=∇×k=∂∂xky-∂∂ykx=0

∂∂xksinθ-∂∂ykcosθ=0

Chain rule yields;

sinθ∂k∂x+k∂sinθ∂x-cosθ∂θ∂y+k ∂cosθ∂y=0

sinθ∂k∂x-cosθ∂k∂y +kcosθ∂θ∂x+ksinθ ∂θ∂y=0

This equation for θ can be simplified for the unique case of straight and parallel offshore contours, in that case

∂∂y=0

ddxksinθ=0  ∴  ksinθ=constant in x

For steady waves σ is also constant so we use a trick and divide through σ

kσsinθ=constant 
sinθC=constant

In deep water C= Co and θ=θo so,

sinθC=sinθoCo

Snell’s Law, governing wave refraction

We know that C decreases in shallow water in order for sinθC to remain constant as C decreases sin(θ) must also decrease or θ must decrease.

The result is that waves become more shore normal as they approach the coast.

Waves Refract as they move into shallow water.

Conceptual discussion:

The reason for the change in wave direction is related to the bathymetric profile of the seafloor.  We know that the wavelength shortens as waves move into shallow water, and in response to this shortening of the wavelength, the wave slows down.  As waves approach the shore at some oblique angle, the landward portion of the wave crest will reach the shallower waters first.  This part of the wave crest will begin to slow down before the rest of the wave that is still travelling over deeper waters.   So as one section of the wave begins to move slower than the rest, the wave begins to turn toward the shore.

We can understand conceptually why this occurs and now we can use the equations given above to calculate the change.

# Conservation of Energy

- Energy travels along wave Rays

- There is no energy flux between/across wave Rays

bo

0

b1

1

Let’s look at 2 wave rays separated by an along crest distance of bo in deep water.

The total average energy flux along that crest length, bo is EoConobo as the wave refracts and moves toward shore.

At location 1 the total average energy flux along the crest length b1 is E1C1n1b1

Conservation of energy dictates that in absence of dissipation, Energy flux is constant,

Fobo= F1b1

EoConobo= E1C1n1b1

Our total average energy flux must be conserved though energy density can change.

Thus, we can solve for the change in wave height:

E=18ρgH2

18ρgHo2Conobo=18ρgH12C1n1b1

H1=HoCg0 Cg1 bob1

H1=HoCg0 Cg1bob1

Refraction Coeff

Kr due to direction change

Shoaling Coef.

Ks due to  speed

b1, perpendicular distance always increases as waves shoal

bo=lcosθo

The perpendicular distance b equals the horizontal distance, l, times cos of the angle, b approaches l  as waves approach coast.

bob1=lcosθolcosθ1     ∴   Kr=cosθocosθ1

H1=HoCg0 Cg1cosθocosθ1

General Form:H2=H1KrKs

H2=H1Cg0 Cg1cosθ1cosθ2

We can solve by directly calculating Cg1 and Cg2 and cosθ2 using dispersion and Snell’s Law or,

Figure 4.18 relates θ, hgT2 to Kr

Figure 4.19 relates θ, hgT2 to KrKs

Change in wave height due to the slowing down of the wave

Shoaling:

Ks     =    Cg0Cg1

As wave slows, L decreases; HOWEVER, Energy = total average energy per unit crest length must be the same!

(Recall: in shallow water, C= Cg, n=1)

E1C1n1= E2C2n2

E1C1= E2C2

18ρgH12L1T1=18ρgH22L2T2

H12L1=H22L2

Conservation of energy flux

At zeroth order:

18ρgH12L1=18ρgH22L2

H12L1=H22L2

Conservation of Average

Energy per unit crest width

H increases as L decreases

Because energy travels at group velocity in intermediate depths we must balance the interplay of n with C in order to find ∆H

From deep to shallow water we find as no= ½  and nsw=1

2Hsw2Lsw=H02Lo      we see a factor of 2 introduced

HswHo=12LoLsw

Hsw=Ho2LoLsw

# Example

Wave H=2m in deep water, propagates toward shore at 30° angle with period of 15 s

Assume straight and parallel contours.

What is the height and direction of 8m water?

Method 1: Solve

θo=30° ,  Ho=2m,    T=15s

Solve for Lo

Lo=gT22π=351 m                                                          ko=0.0179radm

Co=σko=23.4ms                                                                 σ=0.149rads

Cg=nC=11.7ms

In 8m:

L8=129.71 m                                               ko=0.0484radm

C8=8.65ms

Cg8=8.24ms

Snell’s Law       kosinθo=k1sinθ1
 sinθ1=kok1sinθo=0.370sinθo
sinθ1=0.185

θ1=10.70

Energy Cons      EoCgocosθo=E1Cg1cosθ1
Ho2Cgocosθo=H1Cg1cosθ1
H1=HoCgoCg1  cosθocosθ1

=2m 11.78.24 cos30cos10.7=2m1.42 0.881

=2.24 m

Method 2: Use figure 4.19

hgT2= 8m9.81ms215s2=0.0036

θo=30°

θ1≈10.5°,     krks≈1.13 
H1=2m1.13=2.26m

Method 3: Wave refFree app

H=2.0		Lo= 351.29 m

T=15		Co= 23.42 m/s

Θ= 30.0	Cgo=11.71 m/s

h=8

GO		L8=129.71m

C8=8.65 m/s

Cg8= 8.24 m/s

kr= 0.94

ks=1.19	H8=2.24m

Θ8=10.6°

# Diffraction

Spreading of wave energy laterally to direction of wave propagation.

Important in structures and where waves come to together-caustics

Waves ‘bend’ around objects as energy spreads (moves with crest along a ray.)

Refraction and Diffraction often occur together

Direction and height changes due to structures

Direction and height changes due to ∆h

Most structures are in water depths that are shallow enough for wave-bottom interactions.  It is important to calculate together, but difficult.

Typically, diffraction is estimation using nomograms from CEM.

Or more modernly computed:

- REF/ DIF,

- STWAVE,

- SWAN, etc.

# Wave Breaking

2 kinds of Breaking: depth limited and steepness limited. Waves break when the wave height is approximately equal to the wave depth.

As the wave feels the bottom, friction slows the wave speed; however, the particles in the crest do not feel the friction to the same extent as the particles closer to the bottom.

The wave ‘trips’ over the bottom and breaks.

ucrest>C

It is of interest to find the water depth and wave height at the point of breaking.

A waves shoal, we have shown that height increases. This becomes unstable and eventually waves break.

M=tanβ= beach slope where β= angle of the beach slope

# Surf Similarity number – Iribarren Number

In order to determine the extent to which a wave breaks or is reflected off the beach face back offshore, we refer to the Iribarren Number,

ξ0=tanβHoLo

or surf similarly parameter based on the deep water wave height.

Can also base parameter on Breaking wave height just set: H=Hb

ξb=tanβHbLo

ξ0=m2πHogT2   → ξ02=m2gT22πHo
 ξ0=mHLo                                    ξo<0.5<ξo<3.3<ξo

ξb<0.4<ξb<2.0<ξb<4<ξb

Spilling	plunging 	surging	reflection

(no breaking)

3 kinds of depth limited breaking:

- Spilling:		ξo<0.5 or ξb<0.4			HmgT2>0.08

- Dissipative

- Mild sloping beach

- Numerous waves in surf zone

- Gentle breaking, tends to be further from shoreline

- Plunging:	0.5<ξo<3.3 or 0.4<ξb<2.0	0.005<HmgT2<0.08

- Dissipative

- Steeper beaches

- Violent breaking

- Aerated in tube

- Collapsing / Surging:	ξo>3.3 or 2<ξb<4		HmgT2<0.005

- Small waves on steep beach

- Breaks weakly on beach face

- Reflection of wave

For, 4 <ξb , Little or no breaking, wave energy is reflected

# Depth limited breaking

Waves break in water depth approximately equal to the wave height.

Hb=κhb

The proportion κ ≅0.78 (McCowan 1894)

Lab studies showed a dependency of κ on Beach slope m:

κ=bm-a(m)HbgT2

where:

am=43.8(1.0-e-19m)

bm=1.56 (1.0+e-19.5m)-1

As m approaches zero, κ  0.78

From energy conservation shoaling and refraction

Hb=Ho12ConbCbcosθ0cosθb

We can solve this for independent breaking depth. First assume breaking is a depth limited and wave is in shallow water.

Hb=Ho12Co2gh cosθ01

Second assume McCowan’s Breaking condition,
Hb=κhb

κ= 0.78

khb=HoCocosθ02gh

Now solve for hb

k2hb2=Ho2Cocosθ02gh
2ghbhb2=Ho2k2 Cocosθ

hb5=Ho44gk4Co2cosθ02

hb=1g15k45(Ho2Cocosθ0)25415

# Steepness limited Breaking

In deep water, limiting wave steepness is:

HoLomax≈0.17-0.14 ≅17

H is17 h, L is 7 times H

Steepness limited breaking

Minimum sustainable crest angle is 120 degrees

α=120°

# Summary of Linear (Airy) Wave theory- Properties

L=wavelength; k=wave number; h= water depth; σ=Angular wave frequency; T=period; C=wave celerity; Cg=group velocity

| Relative Depth | Shallow water h/L <1/20 | Transitional Water 1/20< h/L <1/2 | Deep Water h/L <1/2 |
| --- | --- | --- | --- |
| Wave Profile | Same as |  | Same as |
| Wave Length |  |  |  |
| Wave Celerity |  |  |  |
| Group Velocity |  |  |  |
| - Horizontal particle Velocity |  |  |  |
| - Vertical particle Velocity |  |  |  |
| - Horizontal Particle acceleration |  |  |  |
| - Vertical Particle acceleration |  |  |  |
| - Horizontal particle displacement |  |  |  |
| - Vertical particle displacement |  |  |  |
| Sub-surface Pressure |  |  |  |

# Wave Statistics and Spectra:

Single frequency waves are called monochromatic referencing light waves.

Up until now we have only been looking at monochromatic waves. We have derived solutions based on single frequency waves.

In reality, the sea surface is a superposition of many waves each having its own frequency, direction and amplitude.

In order to represent the sea surface we must superimpose many waves.

And to understand a sea surface we must decompose the raw elevation signal into the constituent components.

How do we extract the wave properties from a raw signal?

- Wave-by-wave analysis

  - Counting individual waves in a time series signal

  - Analog time series/Plot on paper

- Spectral analysis

  - This decomposition is performed by applying Fourier Transforms to the signal.

  - Digital time series/Numerical calculation

For our wave record the sea surface, η(t) is given by:

ηt=n=0∞ancos(σnt-ϵn)

It is useful to arrange the data such that the researcher can gain insight into the characteristic frequencies of a signal.

The ability to plot the representative energy, a2, vs frequency allows us to determine which frequencies are carrying the most energy in our signal.

Such a plot an2 vs σn is called the energy spectrum. In reality we have waves of different σ and θ. When interested in direction we will calculate the directional wave spectrum.

# Wave-by-wave analysis

Divide up signal into individual ‘waves’

Zero up crossing and zero down crossing

Need heights and periods for each wave

Let’s look at irregular waves:

η=MWL

t

NGVD

η

Superposition of many waves with different heights, frequency, phases, and directions

If we examine η in time

period

t

η

height

We define a wave by height and period.  From the signal we can pick out ‘waves’ by marking successive zero-upcrossings.

So let’s go back to η above and we see that at this location, for this time duration we count 5 waves. Each has its own height, period. We can then collect this discrete data (height and period) and plot as bins (histograms).

In practice we record data from an analog plot then rank order (sort) the waves from largest to smallest, and finally bin the data into histograms.

The histogram can be binned by height

# of waves

Wave height

Statistical measures can now be calculated.

Mean H=1niHi   (average height)

Hrms is always larger than H

Measure of the magnitude of the varying wave height

Root-mean squared wave height     Hrms=1niHi2

Max

Min

Mean  average value

Mode  most common value

Median  half greater, half less

Gives wave height with equivalent energy to the average energy in signal

Rms≡Hrms=1niHi2

Special statistics we like:

H1n= the mean of the heighest 1/n waves

H13=Hs= significant wave height =  the mean height of the highest 1/3 waves

H110=  the mean height of the highest 1/10 waves

H1=H=  the mean height of all waves

If N ≡ total number of waves,

The wave height distribution is defined by the probability.

Probability that a wave height is greater or equal to some value is given by:

PH>H=nN                                          or PH≤H=1-nN

n= number of waves >H

# Discrete Wave Statistics:

H,Hrms, H13      btw H 1/3 ≡significant wave height

Kinsman (1965) determined a strong correlation between significant wave height and the visual wave height determined by the experienced observer.

Period can be defined as time between consecutive crests or zero crossing period (up or down). Now that the signal has been analyzed we can calculate probabilities. [Longuette-Higgins 1952 determined that the random distribution of waves followed known probability laws].

For a single, monochromatic wave signal

η(t)=H2cosσt ; all waves have the same height

∴ Hp=Ho for all P          and Hrms=Ho

If we now add another monochromatic wave of the slightly different phase but the same wave height (as we did for constructing the wave group), we obtain a new signal of waves modulated by an envelope.

To determine Hp. since wave height decreases monotonically from max to min,

We average the wave height average from t=0 to P(π/∆σ)

Hp=4 Hopπsinpπ2        and      Hrms=2Ho

So Hp=22Hrmspπsinpπ2       as p 0, sin(p)p

∴sin(Pπ/2) Pπ/2

As P0, Hp Hmax

Hmax= 2Hrms

And recall Hrms=2H0  ∴Hmax=2H0 as expected!

    - EXAMPLE:

What are Hmax, H110, and H1 in terms of Hrms

Hmax=2Hrms=1.414Hrms

H110 =202Hrmsπsinπ20=1.408Hrms
H13=62πHrmssinπ6=1.350Hrms

H1=22πHrmssinπ2=.9Hrms

Example 2: If you are an experienced observer and observe average wave height at the beach of 1.219m≈4 ft, what is the Hmax and H1?

Well it has been found that observed heights are correlated with H13

∴1.22m=1.350Hrms

Hrms= 0.904m=2.97ft

H1=.813m, 2.67ft

Hmax=1.28m, 4.2ft

# Probability

Probability distribution (Probability mass function) P(x)= This function gives the frequency of occurrence of a possible outcome or value of a random variable.

It is important to note that:

0≤Px and Px=1,

Another way to express probabilities is by the cumulative Probability C(x), this is the fraction of total population of events that a particular event is not exceeded.

Cx=PH≤H=1-nN

σ is stand dev of x

μ is mean of x

Probability Density, p(x): this is the fraction of events in a population that is made up of a particular event. It represents the rate of change of a particular distribution. Or p(x)= dC(x)dx . The two most common probabilities are Gaussian and Rayleigh Densities.

The Gaussian Probability Density is

p(x)=1σx2π e-(x-μx)22σx2

The integral of P(x) is the probability distribution P(x) for a zero mean, μ=0, and unit σ=1,

p(x)=12πe-x22

P(x)=Φx-μxσx=Φ=0xpxdx

Where the integral Φ=0x12π e-x22  is known as the error function, erf(x)

Cumulative distribution

1

Density Function

.5

density

σx

σx

P(x1)

μx

x1

Random variable x

The probability of exceedance Q(x) can be found as Q(x(t)>x1)=1-P (x(t)<x1)=

1-Φx-μxσx  which is the probability that x will exceed x1  over time t.

Gaussian distribution is most useful for short term probabilities of free surface η(x,t).

We are more concerned with wave height H rather than η. Longuet-Higgins (1952) examined statistically η(t) and found that both H and a followed the Rayleigh distribution.

Narrow Banded Spectra: The Rayleigh Distribution

p(x)=πx2μx2e-π4xμx2  for x≥0

P(x)=1-e-π4xμx2  for x≥0

Rayleigh Dist.  is asymmetric about mean

1

1

.55

x1

4α

3α

2α

mean

α

P(x)

If we assume most wave energy falls under a small band of wave periods (narrow banded) the wave height probability density can be determined by

pH=2HHrms2e -HHrms2 or PH=1-e-H2/Hrms2 or P(H>H)

= e-HHrms2

Previously, P(H>H)= n/N for rank ordered grouping

∴nN=e-HHrms2

N is the number of waves onto N total waves that have a height greater than or equal to H

We can rearrange this eq to find the height exceeded by the n largest waves in the group

H=HrmslnnN         Hp=Hrmsln1p ≡ height exceeding pN of the waves

Example: 400 waves in record

N=400

Hrms=1NHi2

- How many waves are expected to exceed H=2Hrms

n=Ne-HHrms2=400e-2HrmsHrms2=400e-22=400e-4=7.3, 7 waves

nN=7400=0.0175 x 100%=1.75% of the total # of waves exceeded 2Hrms

- Height H exceeded by ½ waves (n=N/2 or P=1/2)

H1/2=Hrmsln2= 0.833Hrms

- Height exceeded by one wave? For H1/N we have P=1/N=1/400

H1/400=Hrmsln400=2.45 Hrms

4

As N ↑, Hmax ↑

Since PDF never actually reaches zero, rather decays asymptotically.

H110=1.80Hrms
H13=1.416 Hrms
H1=0.886Hrms

Or in general

HpHrms=ln1p+ π2perfc(ln1p    why?

more…complentary error function

Time series and Statistics (DD 7.1-7.2)

Histograms and Distributions (DD7.1-7.2)

# Spectral Analysis

Any piecewise continuous function can be expressed by a sum of sines and cosines over some interval tot+T

Wave record sampled at regular discrete interval (∆t~0.5s-2s) and finite in length: 20 min – 2 hr

Spectral method will bin record into frequencies ‘automatically’. No need to sort the wave periods first.

Any periodic signal can be represented by an infinite sum of Fourier components:

ηt=n=0∞ancosnσt+ bnsinnσt

In reality we have a limited series not an infinite, N not ∞, so for a finite record

ηt=ao+n=1Nancosnσt+ bnsinnσt

where an=2Ttt+Tftcosnσt dt       even component

bn=2Ttt+Tftsinnσt dt      odd component

and ao= 1Ttt+Tf(t) dt = average for mean value in record

ao (Mean) usually subtracted out

We typically use FFT (Cooley and Tukey,1965) which exploits symmetries to streamline calculations,  to obtain an and bn, ‘fast former transform’

We will not get into theory behind Fourier sums and transforms for this class.

The FFT is a routine available in most canned mathematical programs like MatLab and Mathematica also in ‘numerical recipes’ (Press et al 1986)

FFT is most efficient when N is some power of 2

N= 512, 1024, 2084 or 4096 points

We treat FFT as a ‘black box’ so for us

FFT N an,bn

Time series

Where N is the number of frequesncies returned, anand bn, are the coefficients for each of the N frequencies.

And the inverse to regain time series

N an bn  FFT-1

Time series

Let’s describe a time series at ∆t intervals

So if we have a total record length of T seconds and it is split or digitized into N evenly spaced, ∆t, points:

N∆t=T

Or the total number of samples is,

N=T∆t

Our continuous time record becomes a discreet number of points

f(t) fi, i=1,…,N

σ =2πT

where T is the total time interval

σ =2πN∆t

We can write

ft= n=-NNFneinσt
Fn=an-ibn2  and  σ=2πT=2πNΔt

In a finite discrete form (only N points)

ft=fj=n=0N-1Fnei2πntjN

So if we know ALL Fn from n=0 to N-1 we can find the time record at time j.

We can solve for the nth coefficient Fn by:

Fn=1Ttt+Tft e-inσtdt=1T0Tfte-2πintN∆tdt=an2-ibn2

or for F-n=Fn*=an2+ibn2

These are the finite discrete forms of the former transform pairs

We can use the equation above for Fn, to calculate an & bn, however typically we will use FFT to do this.

Inputting a time series (N values) we get back N an  , N bn , one for each N frequency.

Our frequency resolution for the lowest frequency that can be resolved (fundamental frequency or Bandwith), Δf or f0, is determined by the ∆t and number of records.

∆f=f0=1T=1N∆t

For a fixed N and a finer sampling rate will lead to a larger ∆f, or higher frequency resolution.

For a ‘good’ representation of infinite series using a finite record an & bn, we need a large enough record length, N.

But how big is big enough?

How do we know if we have a long enough wavelength record?

If we take the average mean square value of function f :

1T0t+Tf2tdt

(average mean square value)

½ the sum of the squares of the Fourier coefficient should equal approx.:

ao2+12n(an2+bn2)≈ 1Ttt+Tao+n(ancosnσt+bnsinnσt2dt=1T0t+Tf2tdt

Parseval’s Theorm

(Often de-mean the signal to get rid of ao)

If not, then we need to add more numbers to the series, make N bigger.

What do an & bn look like?

Most output is ordered from longest frequency to N∆f

an

N∆f

(N/2)∆f

f1

bn

N∆f

(N/2)∆f

f1

a’s are exactly symmetric about (N/2)∆f, while the bn are skew-symmetric about (N/2)∆f.

The results are typically presented by plots

Fn2=an42+bn42

Results in a two sided spectrum

Fn2

1

N∆f

(N/2)∆f

Which is then typically ‘folded’ about (N/2)∆f.

2Fn2=an2+bn22

(N/2)∆f

Results in a spectrum - a distribution of energy with frequency.

Most energy is contained in the spectral peak.

We can calculate the spectral density (or energy density).

Sf=2Fn2∆f

For a free surface time series, in meters S(f) has units of m2s or m2/Hz

The variance σ2 =[ηt]2    (mean square of the surface elevation)

Let’s connect the energy density spectrum to the energy of a wave.

Assume: η(t)= a sinσt

Calculating variance over 2π

σ2=[ηt]2=12π02πa2sin2σt dσt=a22

a22=20∞Sfdf  for a monochromatic wave.

The variance, wave energy and wave energy spectrum are related.

Given an energy spectrum S(f), the wave amplitude for a given frequency is

af=2S(f)∆f

For monochromatic wave show fig 6.11 pg  175

For a random wave show fig 6.12 pg 176

Directional spectra, 6.13, 6.14

The limiting frequency at which a wave can be adequately resolved is called the Nyquist frequency, or folding frequency

fNyq=12∆t

T=2∆t       σ=π/∆t

For any higher frequency waves, (T smaller), the sampling rate is inadequate.  Researcher must design ∆t to avoid aliasing.

## Directional Spectra

In nature waves travel in all directions, as they are generated all over the planet.  In order to better understand which direction sends the most wave energy to a location of interest, we can look at the directional spectrum.  Peaks in the directional spectrum indicate the dominant location from which the most energy is coming.

Sf,θ=Sf∙Dθ

Sf=θ=θminθmaxSf,θ∆θ

Dθ=N=1NSn(F,θ)∆f

For directional spectrum (wave energy as a function of frequency and direction)

af,θ=2S(f,θ)∆f∆θ

In depth details about directional spectra are left for a more advanced course on wave mechanics.

# Spectral moments

The power density plot has a lot of information contained within.  We have the Energy in the form of wave amplitude squared and period in the form of frequency.  By analyzing the spectral density we should be able to tease out pertinent characteristics.

One method of achieving this goal, is through the calculation of spectral moments.  Given the formula for spectral moments:

mi=0∞fiSfdf=n=0NfniSfn∆f            i=0,1,2…

For Rayleigh Dist Waveheights (L-H 1992) determined

H13=Hs~Hm0=4mo=4n=1NS(fn)∆f

Or 4 times the sqrt of the area under the curve S(f)

For Directional spectra:

H1/3=Hs~4mo=4n=1Np=1PS(fn,θp)∆θ∆f

We can also calculate mean wave period Tmo1

T=2πm0m1
CHECK THIS

f=m1m0

TMo1=1NfSf∆f1NSf∆f-1
Tmo2=1Nf2Sf∆f1NSf∆f-1

Peak Period= 1/fp   frequency of spectral plate

mo≡ area under spectral curve (power in signal) so m0≡(H)

Signal with f   G has

2m2mo zero crossings/sec

So for Tmean=Tmo2≡mom2

Some other parameters that may be found about a sea state though its energy spectrum include number of zero up-crossings (N0), the average zero crossing period (), The average apparent wavelength () and the average crest to crest period ().

No of zero upcrossings			 (s-1)

Average zero crossing period		 (s)

Average apparent wave length		 (m)

Average crest to crest period		 (s)

(Mean) Characteristic Period		(s)

FFT and Wave Spectra II (DD7.3-7.4)

Nomenclature

H1/10		Average of the highest 1/10 waves

H1/100		Average of the highest 1/100 waves

Hmax		Maximum wave height

Hs   H1/3	Significant wave height

Uw10		Wind speed at 10m above SWL.

Uw19.5		Wind speed at 19.5m above SWL

X		Fetch length.

Sea surface elevation

Spectral bandwidth

m0		Zero Moment (area under spectrum).

mn		nth moment of area under the spectrum.

B		Dimensionless constant used for the Neumann spectrum

Average Crossing period

ABBREVIATIONS

CF 		Correction Factor

FFT		Fast Fourier Transform

ISSC		International Ship Structures Congress

ITTC			International Towing Tank Conference

JONSWAP	Joint North Sea Wave Project

P-M		Pierson – Moskowitz spectrum

SWL		Still Water Level

## Wave Energy Spectrum

If the ocean is observed at a particular point on a short term basis it may be assumed to consist of numerous waves of different periods, propagation directions and height. Due to this idealisation to analyse random seas in terms of a regular sinusoidal waveforms spectral analysis may be employed. A wave spectrum will analyse a sea state in the frequency domain and describe the energy transmitted by the wave conditions at a given frequency. However, when the wave data is collected it will usually be done so in terms of the surface elevation (η) in the time domain and therefore must be converted into the frequency domain. Also, the continuous wave system must be calculated at discrete intervals. The method used to perform this conversion is the Fast Fourier Transform (FFT). For a more detailed discussion on FFT refer to Chakrabarti (1987).

Ocean waves are primarily influenced by the speed and directionality of the wind and as such these are also the determining factors when describing spectra. The wind speed used to calculate the spectra will be taken as Uw19.5 however Uw10 may also be used in some cases. The directionality component of the wind will also encapsulate the fetch length (X), which is the length that the wind will blow to propagate a wave. As the wind blows over the surface of the sea waves will continue to grow until the point at which they break, thus limiting their height. At this point, ignoring the effect of swells, the sea is said to be fully developed and thus will be in a relatively steady condition. A large area and propagation time are required to produce fully developed seas. In many parts of the world it is more common to have either fetch limited or duration limited conditions. Fetch limited seas are seen when fetch is the limiting factor in producing fully developed seas. Duration limited seas are evident when the wind does not blow for long enough to produce the fully developed conditions.

Once a spectrum has been established it must be smoothed to eliminate the effects of noise (Chakrabarti 1987). This is done by averaging it over frequency ranges longer than the finite elements described in the Fourier method (Kamphuis 2000). The effect of smoothing the spectra will not change its total energy but it will eliminate the numerous peaks either side of the main peak which are referred to above as noise.

It is often important to refer to moments of a wave spectrum, which are defined as:

This is because they enable easy interpretation of the energy spectrum and easy manipulation to give meaningful outputs. For example the zero moment (m0) is representative of the area under the spectrum and once it is known the wave heights may be determined as follows;

(note: The units are dependent on the input data)

Significant wave height, 			(m)

Average of the highest 1/10th,			(m)

Average of the highest 1/100th,		(m)

Root mean square of wave height

Since the formulas above are derived assuming Rayleigh distribution which is for narrow frequency spectrum, they will produce erroneous results when applied to broad banded ocean wave spectra. This problem can be overcome through the use of a CF;

Where;

ε is also known as spectral bandwidth and for a narrow bandwidth where ε approaches 0 the wave train is said to remain almost the same. Now that the CF is known it can be applied to the formula for wave height as well as the following formula to provide accurate results for real ocean conditions.

Significant wave height, 			(m)

Average of the highest 1/10th,		(m)

Average of the highest 1/100th,		(m)

Root mean square of wave height

Some other parameters that may be found about a sea state though its energy spectrum include number of zero up-crossings (N0), the average zero crossing period (), The average apparent wavelength () and the average crest to crest period ().

No of zero upcrossings		 (s-1)

Average zero crossing period		 (s)

Average apparent wave length	 (m)

Average crest to crest period		 (s)

(Mean) Characteristic Period			(s)

It is also possible to manipulate the data in the wave energy spectrum to produce useful information relating to ships and other floating structures that are present at that point in the ocean.

## Details on Six Commonly Used Spectrum

### Neumann Spectrum

The first and simplest analytical model of ocean wave spectrum is the Neumann spectrum (1953). It is a single parameter spectrum based on wind speed, Uw, which can be defined as;

Where;

Hs = Significant wave height

### Pierson – Moskowitz Spectrum

In 1964 Pierson and Moskowitz proposed a new type of spectrum. The P-M spectrum is defined as a single parameter spectrum that can be applied to fully developed seas, where the peak frequency occurs somewhere in the middle of the sampling time (CEM 2002). To use the P-M spectrum the wind fetch and duration are assumed to be infinite. The wind must also be assumed to be blowing over a large area, greater than 5000 wave lengths on either side, and from a nearly constant direction. If all these conditions apply then a P-M model may be used to generate the spectrum where Uw19.5 will be the only variable. Due to the conditions that must be applied to the P-M spectrum its is usually limited to offshore and severe storm applications. The P-M spectrum is described as follows;

Where;

### JONSWAP Spectrum

This spectral model was developed by Hasselman et al (1973). It gets it name from the Joint North Sea Wave Project, hence JONSWAP. For development of the constants used in the spectrum wave data was collected over a large area extending into the North Sea from Sylt Island (Delft University Notes). The spectrum was developed as a result of the need to model seas produced by winds of limited fetch and duration. It is a two parameter spectrum based on the P-M spectrum. In reality there are five parameters that define the JONSWAP spectrum, these are γ, τ, ω0, X and Uw. However it is more usual that only two of the parameters, Uw and X, are left as variables. The following formula defines the JONSWAP spectrum;

Where;			γ = peakedness parameter

This is described as the ratio between the peak frequencies of the JONSWAP and P-M spectrum (Figure 2) and can range between 1 and 7. However it is more commonly given as γ=3.3.

τ = shape parameter. This varies from τ = 0.07 for ω≤ω0 and τ = 0.09 for ω>ω0.

X = Fetch. This is one of the variable parameters determined by the operator.

Uw = Wind speed, also determined by the operator.

If the fetch (X) is unknown then α = 0.0081 may be used instead.

It is important to note that the moments (mn) of the JONSWAP spectrum cannot be calculated analytically but may be estimated by numerical integration (Reeve 2004).

### Bretschneider Spectrum

The Bretschneider spectrum is another example of a two parameter spectrum, its parameter relate to wave height and period. The development of the Bretschneider spectrum is based on the assumption that the spectrum is “narrow banded and the individual wave heights and wave periods follow the Rayleigh distribution” (Chakrabarti 1987). The Bretschneider spectrum may be applied to fully developed seas, seas that are 90% developed or seas that are 80% developed. This report, and the accompanying Microsoft Excel spreadsheet, will only deal with the Bretschneider spectrum for fully developed seas. The formula for fully developed seas is;

Where;

Ts = Significant wave period, defined as the average period of the significant waves.

Figure 2: Peakedness Parameter (CEM 2002)

Also, it can be shown that Ts = 0.946T0 where T0 is the peak period. This makes the Bretschneider and P-M spectral models equivalent.

- Long Waves

  - Long Wave Theory

  - Long Wave Equations of Motion

  - Hydrostatic approximation

  - Seiching

  - Storm Surge

- Nonlinear Properties for Small-amplitude Waves

  - Mass Transport

  - Mean Water Level

  - Momentum Flux

  - Wave Forcing

- Wave Forces

  - Intro and Flow around a Cylinder

  - Morison Equation and total force calculation

- Nonlinear Waves

  - Stokes Theory

- Extreme value analysis

  - Extreme value:

Extreme value: the largest value expected to occur in a given or specified number of observations.

Establish how many observations that you would expect to have over a given time period, and then determine the largest predicted value for that number of observations.

- FINAL EXAM